Intent-Driven AI-Native Precise Positioning-as-a-Service

Objective: To demonstrate an intent-driven AI-native precise positioning system delivered through ATSC 3.0 broadcast, where operator or service-provider intents such as maximize accuracy, ensure high reliability in challenging environments, or optimize ATSC 3.0 spectrum usage are automatically translated by the AI agent into real-time configurations of the positioning pipeline.

AI native: This PoC is AI-native because it uses intent-driven orchestration [FGAINN-I-097-R1], where high-level operator intents, such as achieving sub-10 cm accuracy, maximizing reliability, or minimizing ATSC 3.0 spectrum usage are automatically translated by the AI agent into real-time configuration of broadcast PLPs, RTK/bitmap correction parameters, and edge processing policies.

The system integrates:

- ATSC 3.0 broadcast delivery of GNSS correction data, maps, and bitmap-based positioning information

- RTK-style high-precision corrections via broadcast + optional unicast fallback

- AI-driven adaptive control of correction rate, message type, edge compute, and bandwidth allocation

The goal is to achieve:

- 3 to10 cm accuracy

- Robust reliability even in rural or obstructed zones

- Highly efficient spectrum use on ATSC 3.0 broadcast PLPs

https://github.com/tomojitakasu/RTKLIB

Phase 0 — Prep & data

Objective: get tools, sample data, and baseline running.

Tasks:

Install RTKLIB and confirm you can run a basic RTK solution on sample GNSS logs. Deliverable: .pos output files and a short report of baseline errors (no broadcast corrections).

Collect or synthesize RTK correction frames (RTCM) — RTKLIB sample RTCM or synthetic generator.

Create small bitmap tiles for a small area (e.g., 100×100 binary tiles representing “good correction coverage” or landmark masks).

Tip: Use small synthetic scenarios to start (single road, a few tiles).

motivation:

- A broadcast-based correction delivery reduces per-vehicle bandwidth/redundancy compared to unicast — making RTK for many vehicles more efficient.

- The system can support “mass-market” deployment of high-precision positioning for many vehicles, overcoming limitations of conventional NTRIP/unicast correction delivery.

- Broadcast corrections + appropriate encoding and error robustness can maintain sufficient correction quality across a range of receivers, even under mobility and varying reception conditions.

2. Key Performance Indicators (KPIs)

| KPI | Metric |
| --- | --- |
| Positioning Accuracy | Horizontal/vertical error |
| Reliability | Valid correction availability |
| Correction Delivery Latency | Broadcast → UE decode |
| ATSC 3.0 Spectrum Efficiency | Bits/s/Hz for correction PLP |
| Service Continuity | Dropout rate (rural/urban canyon) |

3. System Setup (ATSC 3.0 + RTK + Bitmap)

3.1 Reference Station Network (GNSS Ground Segment)

• Network of multi-GNSS (GPS, Galileo, etc.) reference receivers
• Continuously computes and outputs:
– RTK corrections (RTCM / MSM)
– Bitmap/grid-based error models (ionospheric, tropospheric, multipath probability tiles)
– SSR data for wide-area fallback
• Feeds correction data to the Broadcast Core with strict latency stamps

3.2 Broadcast Core

• Hosts the Intent-Driven AI-Native Positioning Orchestrator
• Responsibilities:
– Interprets high-level operator intents (accuracy, coverage, spectrum efficiency)
– Selects and configures ATSC PLPs for positioning
– Chooses optimal RTK vs SSR ratio
– Adjusts bitmap tile resolution and update frequency
– Controls redundancy, FEC, and repetition for robustness
– Coordinates compute distribution between cloud and Broadcast Edge
• Publishes packaged RTK + bitmap + integrity data to the Broadcast RAN

3.3 Broadcast RAN

• ATSC 3.0 transmitter creates dedicated PLPs for positioning:
– PLP-A: RTK correction stream
– PLP-B: Bitmap/grid-based error maps + integrity
– Optional PLP-C: Low-power IoT receivers or extended coverage
• Broadcasts all positioning content over ATSC RF to UEs and Broadcast Edge nodes

3.4 Broadcast Edge

• Performs localized enhancement and preprocessing, such as:
– Multipath prediction based on environment
– Local correction smoothing / gap filling
– Cache and refine bitmap tiles for low-latency access
– UE-specific fusion optimization policies
• Receives instructions from the AI Orchestrator for local processing parameters

3.5 UE / Receiver Layer (GNSS + ATSC Device)

• Dual-receiver UE with:
– GNSS chipset capable of RTK precision
– ATSC 3.0 demodulator for receiving PLPs
• Software stack includes:
– Bitmap/grid decoder
– RTK/SSR correction consumer
– AI-assisted fusion engine combining GNSS + IMU + broadcast corrections
• Produces high-accuracy position estimates even in weak-signal or high-mobility conditions

4. AI Agent Design

Inputs

Eg Intents:

- “Provide < 3cm accuracy”

- “Minimize ATSC spectrum usage”

- “Guarantee service continuity in weak-signal regions”

- “Support high-speed mobility”

Network & Environmental State:

- ATSC PLP load, SNR, FEC performance

- GNSS satellite visibility & geometry

- Multipath intensity (urban canyon)

- UE velocity

- Latency of reference station feeds

Quality Metrics:

- Correction age

- Position residuals

- Integrity indicators

- PLP packet loss

Outputs (AI-Controlled Parameters)

AI agent autonomously optimizes:

ATSC 3.0 Broadcast Parameters

- PLP bandwidth & FEC rate

- Bitmap tile resolution (coarse vs fine grid)

- Correction update rate (1–10 Hz)

- RTK vs SSR hybridization ratio

- Redundancy / repetition patterns

Positioning Algorithm Parameters

- Fusion weights (GNSS + IMU + corrections)

- Multipath mitigation thresholds

- Station selection / reference diversity

- Edge vs cloud processing decisions

5. Test Scenarios

Scenario 1: High-Accuracy Drone in Rural Area

Intent: “Guarantee sub-10 cm accuracy.”
AI increases correction rate, uses high-resolution bitmap tiles, boosts PLP reliability via stronger FEC.

Scenario 2: Low-Bandwidth Rural ATSC Coverage

Intent: “Optimize ATSC spectrum use.”
AI decreases correction bitrate, switches to lower-rate SSR/bitmap encoding, reduces PLP bandwidth.

Scenario 3: Urban Canyon Environment

Intent: “Maximize reliability.”
AI strengthens error grid resolution, bumps integrity updates, adjusts multipath prediction and fusion weighting.

Reference Architecture

### A. Reference ATSC 3.0 documents

| Required | Why |
| --- | --- |
| A/331 | To deliver positioning data (bitmap, RTK corrections) over ROUTE/DASH datacast. |
| A/300 | For service creation & delivery model. |
| A/322 | If you want to vary PLP robustness for “maximize accuracy” intent. |

### B. Positioning-specific info you need

- RTK correction format (RTCM, MSM messages)

- Bitmap/feature-map data format

- Timing info: ATSC L1 signaling timestamps

- GNSS → Broadcast synchronization mapping (frame alignment)

### C. AI-intent translation

Example intents:

- maximize accuracy → higher bitrate, more frequent RTK updates

- maximize reliability → robust PLP, lower code rate

- optimize bandwidth → compress positioning deltas, reduce frequency

### D. Required APIs

- Generating ROUTE/DASH for non-video data

- Setting PLP parameters (if using a modifiable transmitter)

- AI agent config control for the pipeline

For PoC your codebase will include:

### 1. Broadcast-side modules

- ALP encapsulator (use libatsc3 or write Python/C++ wrapper)

- ROUTE/DASH packager

- Service (SLT/LLS) descriptor generator

- Transmitter control API (if dynamic PLP/power needed)

- AI agent (Python preferred)

  - Intent parser

  - Decision engine

  - Optimizer

  - Controller for ATSC parameters

### 2. Receiver-side modules

- ALP + ROUTE + DASH client
(libatsc3 or your own lightweight Python client)

- Positioning client

### 3. Orchestration layer

- REST/gRPC API exposing intents

- Real-time analytics module