A

PROJECT REPORT

ON

Intent Driven AI-Native Precise Positioning-as-a-Service

Submitted in the partial fulfillment for the award of Bachelor of Technology degree of Manipal Institute of Technology Bengaluru

2025-2026

Guided by:						           Submitted by:

Dr. Shweta Singh   	       			          Rishiraj Rakesh Kumar

(Assistant Prof.)                     				B. Tech. VIII SEM, ECE

Department of ECE	                                         Roll No.: 225807062

DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

MANIPAL INSTITUTE OF TECHNOLOGY, BENGALURU

MANIPAL ACADEMY OF HIGHER EDUCATION

May, 2026

CERTIFICATE

This is to certify that the project titled “Intent Driven AI-Native Precise Positioning-as-a-Service” is the bonafide work carried out by “Rishiraj Rakesh Kumar”, a student of B.Tech. (ECE) of Manipal Institute of Technology Bengaluru, Manipal Academy of Higher Education Manipal India during the academic year 2025-26, in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology (Electronics and Communication Engineering) and that the project has not formed the basis for the award previously of any other degree, diploma, fellowship or any other similar title.

(Signature of the Guide)

Dr. Shweta Singh

(Assistant Prof.)

Department of ECE

ACKNOWLEDGEMENT

I would like to extend my deepest thanks to my project guide, Dr. Shweta Singh, Department of Electronics and Communication Engineering, Manipal Institute of Technology Bengaluru, for her invaluable guidance, patience, and constructive feedback throughout the duration of this project. Her insights consistently helped me refocus on what mattered most and her encouragement during challenging phases of the implementation kept the work on track.

I am grateful to the Department of Electronics and Communication Engineering and the faculty members of Manipal Institute of Technology Bengaluru for providing an academic environment that encouraged independent thinking, hands-on exploration, and technical rigour. The foundation built through the coursework of this programme directly enabled the breadth of this project.

TABLE OF CONTENTS

Certificate………………………………………………………………………………………………………………………………… II

Acknowledgements…………………………………………………………………………………………………………………. III

Table of Contents……………………………………………………………………………………………………………………... IV

List of Figures…………………………………………………………………………………………………………………………... VIII

List of Symbols and Abbreviations………………….………………………………………………………………………… IX

List of Tables……………………………………………………………………………………………………………………………. XII

Abstract……………………………………………………………………………………………………………………………………. 1

1. Introduction…………………………………………………………………………………………………………………………. 2

1.1 General………………………………………………………………………………………………………………….. 2

1.2 Motivation………………………………………………………………………………………………………………. 3

1.3 Problem Statement…………………………………………………………………………………………………. 3

1.4 Objectives………………………………………………………………………………………………………………… 4

1.5 Scope and Limitations………………………………………………………………………………………………. 5

2. Literature Review……………………………………………………………………………………………………………………. 6

2.1 Introduction……………………………………………………………………………………………………………… 6

2.2 Standard GNSS and RTK Positioning…………………………………………………………………………… 6

2.4 RTCM Correction Formats and Interportability…………………………………………………………… 7

2.5 ATSC  3.0 as a Broadcast Delivery Platform…………………………………………………………………. 7

2.6 Machine Learning for GNSS Perfformance Enhancement…………………………………………… 10

2.7 Intent-Driven and AI-Native Network Management……………………………………………………. 11

2.8 Identified Gaps in the Literature…………………………………………………………………………………. 12

3. System Architecture and Design………………………………………………………………………………………………… 12

3.1 Introduction………………………………………………………………………………………………………………. 12

3.2 Top-Level System Overview……………………………………………………………………………………… 13

3.3 Closed-Loop Control Architecture…………………………………………………………………………….. 15

3.4 GNSS Layer Design…………………………………………………………………………………………………….. 16

3.4.1 RTCM Correction Generator……………………………………………………………………….. 16

3.4.2 Coverage Map and BitMap Tile Generator………………………………………………… 18

3.4.3 Scenario Simulator……………………………………………………………………………………… 18

3.5 Data Formats and Interface Specifications……………………………………………………………… 19

3.6 Broadcast Processing Layer Design………………………………………………………………………………20

3.6.1 ATSC 3.0 Standards Alignment………………………………………………………………………20

3.6.2 Physical Layer Pipe Architecture………………………………………………………………… 21

3.6.3 Broadcast Controller Operating Modes………………………………………………………. 21

3.7 AI Orchestration Layer Design…………………………………………………………………………………….. 22

3.7.1 Intent Parser Architecture………………………………………………………………………….. 22

3.7.2 Design Model Architecture…………………………………………………………………………..23

3.7.3 Fallback Policy Design…………………………………………………………………………………. 23

3.8 Software Architecture and Repository Structure………………………………………………………… 23

4. Implementation…………………………………………………………………………………………………………………………25

4.1 Introduction………………………………………………………………………………………………………………..25

4.2 GNSS Correction Generation Subsystem………………………………………………………………………25

4.2.1 RTCM Generator………………………………………………………………………………………….25

4.2.2 Coverage Map Generator…………………………………………………………………………….27

4.2.3 Scenario Simulator……………………………………………………………………………………….27

4.2.4 RTK Evaluation Engine………………………………………………………………………………….28

4.3 ATSC 3.0 Broadcast Processing Subsystem…………………………………………………………………. 29

4.3.1 ALP Encoder…………………………………………………………………………………………………29

4.3.2 FEC Encoder…………………………………………………………………………………………………30

4.3.3 OFDM Modulator…………………………………………………………………………………………30

4.3.4 Frame Builder………………………………………………………………………………………………30

4.3.5 Channel Simulator………………………………………………………………………………………31

4.3.6 Decoder……………………………………………………………………………………………………….31

4.3.7 Broadcast Controller…………………………………………………………………………………….31

4.3.8 RTCM-to-Broadcast Handoff………………………………………………………………………32

4.4 AI Orchestration Subsystem…………………………………………………………………………………………33

4.4.1 Data Preprocessor……………………………………………………………………………………….33

4.4.2 Intent Parser………………………………………………………………………………………………33

4.4.3 Decision Model……………………………………………………………………………………………34

4.4.4 Inference Engine………………………………………………………………………………………….36

4.4.5 Feedback Loop Controller…………………………………………………………………………….36

5. Experimental Setup and Dataset…………………………………………………………………………………………………37

5.1 Introduction………………………………………………………………………………………………………………..37

5.2 Software Environment…………………………………………………………………………………………………37

5.3 Three Canonical Test Scenarios……………………………………………………………………………………38

5.3.1 Scenario 1 – High-Accuracy Open Sky…………………………………………………………..38

5.3.2 Scenario 2 – Low-Bandwidth Rural ATSC Coverage………………………………………..39

5.3.3 Scenatio 3 – Urban Canyon Environment……………………………………………………..40

5.4 RTKLIB Configuration Profiles………………………………………………………………………………………41

5.5 AI Model Training Setup………………………………………………………………………………………………42

5.6 KPI Definitions and Measurement Methodology………………………………………………………….43

5.7 Limitation of Experimental Setup……………………………………………………………………………….45

6. Results and Discussion……………………………………………………………………………………………………………….46

6.1 Introduction………………………………………………………………………………………………………………..46

6.2 Scenario 1 – High Accusracy Open Sky…………………………………………………………………………46

6.2.1 Positioning Result………………………………………………………………………………………..46

6.2.2 Broadcast Pipeline Results……………………………………………………………………………48

6.3 Scenario 2 – Low Bandwidth Rural ATSC Coverage……………………………………………………….49

6.3.1 Broadcast Pipeline Results……………………………………………………………………………49

6.3.2 Positioning Results……………………………………………………………………………………….51

6.4 Scenario 3 – Urban Canyon Environment……………………………………………………………………..52

6.4.1 Traditional Baseline Results………………………………………………………………………….52

6.4.2 AI-Native Results………………………………………………………………………………………….53

6.5 Intent Fulfilment Analysis…………………………………………………………………………………………….54

7. Conclusion and Future Work……………………………………………………………………………………………………..55

7.1 Conclusion………………………………………………………………………………………………………………….55

7.2 Future Work……………………………………………………………………………………………………………….56

8. References…………………………………………………………………………………………………………………………………58

LIST OF FIGURES

Figure 3.1: Top-level architecture of the Intent-Driven AI-Native PPaaS platform showing the five logical layers and data flow from GNSS reference station to vehicle receiver and back through the AI feedback loop……………………….............................................................................................13

Figure 3.2: Sequence diagram of the closed-loop broadcast-assisted RTK pipeline. The base station generates RTCM corrections that are packaged, FEC-protected, and broadcast over ATSC 3.0. Vehicles apply corrections in the RTK engine and report aggregated telemetry back to the AI controller, which updates the broadcast configuration to maintain positioning performance……………….………………………………………………………………………………………………….15

Fig. 3.3: RTCM 3.x binary frame structure. The preamble byte (0xD3) marks frame start, followed by a six-bit reserved field and a ten-bit payload length, a twelve-bit message type identifier, the variable-length payload, and a three-byte CRC-24Q for error detection………………………….17

Fig. 3.4: Bottom — Vehicle telemetry JSON record containing RTK solution metrics, environment flags, and vehicle state. Top — AI broadcast command JSON showing the output of the orchestrator including intent, broadcast configuration parameters, reasoning, and confidence score…19

Fig. 3.5 Project repository directory structure showing the three primary module groups and integration layer………………………………………………………………………………………………………………24

LIST OF SYMBOLS AND ABBREVIATIONS

AI	:	Artificial Intelligence

ALP	:	ATSC Link-Layer Protocol

ADAS	:	Advanced Driver Assistance Systems

ATSC	:	Advanced Television Systems Committee

BCH	:	Bose–Chaudhuri–Hocquenghem

BKG	:	Federal Agency for Cartography and Geodesy

BRR	:	Byte Recovery Rate

CGSIC:	Civil GPS Service Interface Committee

DOP	:	Dilution of Precision

DGNSS:	Differential Global Navigation Satellite System

ECEF	:	Earth-Centred Earth-Fixed

ESA	:	European Space Agency

FEC	:	Forward Error Correction

FFT	: 	Fast Fourier Transform

FIX	:	RTK Fixed Integer Ambiguity Solution

FLOAT:	RTK Floating Ambiguity Solution

FG-AINN:	Focus Group on AI-Native Networks

GNSS	:	Global Navigation Satellite System

GPS	:	Global Positioning System

GI	:	Guard Interval

HPE	:	Horizontal Position Error

IMU	:	Inertial Measurement Unit

IFFT	:	Inverse Fast Fourier Transform

IP	:	Internet Protocol

ITU-T	:	International Telecommunication Union — Third

KPI	:	Key Performance Indicator

LDPC	:	Low-Density Parity-Check

LLS	:	Low-Level Signalling

MAE	:	Mean Absolute Error

MC	: 	Monte Carlo

ML	:	Machine Learning

MLP	:	Multi-Layer Perceptron

MSE	:	Mean Squared Error

MSM	:	Multiple Signal Message

NAB	:	National Association of Broadcasters

NLOS	:	Non-Line-of-Sight

NTRIP	:	Networked Transport of RTCM via Internet Protoco

OFDM	:	Orthogonal Frequency-Division Multiplexing

ONNX	:	Open Neural Network Exchange

OODA	:	Observe–Orient–Decide–Act

PAPR	:	Peak-to-Average Power Ratio

PLP	:	Physical Layer Pipe

PPaaS	:	Precise Positioning as a Service

PPP	:	Precise Point Positioning

QAM	:	Quadrature Amplitude Modulation

QPSK	:	Quadrature Phase-Shift Keying

RAN	:	Radio Access Network

RINEX:	Receiver Independent Exchange Format

RMS	:	Root Mean Square

RS	:	Reed-Solomon

RTCM	:	Radio Technical Commission for Maritime Services

RTK	:	Real-Time Kinematic

SE	:	Spectral Efficiency

SLT	:	Service Layer Table

SNR	:	Signal-to-Noise Ratio

SSR	:	State-Space Representation

TTFF	:	Time to First Fix

UAV	:	Unmanned Aerial Vehicle

UE	:	User Equipment

VPE	:	Vertical Position Error

:	Standard Deviation

LIST OF TABLES

Table 3.1 Mapping of broadcast pipeline modules to ATSC 3.0 standard documents……………………20

Table 3.2 Three canonical intents with default constraint thresholds……………………………………………22

Table 5.1 Software environment for experimental evaluation………………………………………………………37

Table 5.2 AI model training hyperparameters……………………………………………………………………………….42

Table 5.3 KPI definitions, measurement methods, and target values…………………………………………….44

Table 6.1 Scenario 1 positioning metrics — Traditional baseline (V4)……………………………………………47

Table 6.2 Scenario 1 positioning metrics — AI-Native optimised (V5)……………………………………………47

Table 6.3 Scenario 1 broadcast configuration —AI-Native (V5) under Maximize Accuracy intent…..48

Table 6.4 Scenario 2 broadcast configuration and performance comparison…………………………………50

Table 6.5 Scenario 2 positioning metrics summary……………………………………………………………………….51

Table 6.6 Scenario 3 positioning metrics — Traditional baseline (V4)……………………………………………52

Table 6.7 Scenario 3 broadcast configuration — AI-Native (V5) under Maximize Reliability intent..53

Table 6.8 Scenario 3 broadcast performance — Traditional vs AI-Native……………………………………….54

Table 6.9: Intent fulfilment assessment per scenario…………………………………………………………………….54

ABSTRACT

This project presents the design and software implementation of an Intent-Driven AI-Native Precise Positioning-as-a-Service (PPaaS) platform that delivers Real-Time Kinematic (RTK) GNSS corrections over an emulated ATSC 3.0 broadcast pipeline. The system addresses two fundamental limitations of conventional correction delivery: the per-receiver scalability constraints of unicast NTRIP-based systems, and the inability of static broadcast configurations to adapt to spatially and temporally varying signal environments. The platform integrates a GNSS correction generation pipeline producing RTCM 3.x binary frames, an ATSC 3.0-compliant broadcast processing chain implementing ALP encapsulation and OFDM modulation across configurable Physical Layer Pipes, and an AI orchestration layer comprising a natural-language intent parser, a residual neural network decision model with Monte Carlo Dropout uncertainty quantification, and a closed-loop feedback controller. Evaluation across three canonical scenarios open-sky rural, low-bandwidth rural, and urban canyon demonstrates a 10 percentage-point improvement in RTK FIX availability, a 5.9% spectral efficiency gain under bandwidth-constrained operation, and a 45.6% improvement in broadcast byte recovery under urban canyon conditions. The complete implementation uses exclusively open-source tools at zero licence cost.

CHAPTER-1

INTRODUCTION

1.1 GENERAL

The proliferation of autonomous mobility systems encompassing ground vehicles, unmanned aerial systems, and precision logistics platforms has fundamentally elevated the accuracy requirements placed upon satellite-based positioning infrastructure. Where legacy applications tolerated positioning errors measured in metres, modern safety-critical deployments such as Advanced Driver Assistance Systems (ADAS), cooperative multi-vehicle navigation, and autonomous drone operations demand sub-decimetre and, in many corridors, centimetre-class positioning with high continuity guarantees. The Global Navigation Satellite System (GNSS), in its standard single-point positioning mode, delivers horizontal accuracy on the order of five to ten metres under open-sky conditions a performance envelope that is entirely insufficient for lane-level vehicle control or precision UAV landing operations.[1]

Real-Time Kinematic (RTK) positioning elevates GNSS accuracy to the centimetre level by exploiting carrier-phase observables alongside a stream of correction messages derived from a network of reference stations with precisely known coordinates. [2] When corrections are fresh, the geometry is adequate, and the signal environment is benign, RTK receivers routinely achieve horizontal position errors of one to three centimetres.[2] However, RTK performance is strongly conditional on the reliability and latency of the correction delivery channel. In conventional deployments, corrections are streamed to individual receivers over unicast IP connections using the NTRIP protocol (Networked Transport of RTCM via Internet Protocol). While this approach is technically effective in controlled settings, it introduces well-documented scalability constraints: bandwidth demand grows linearly with fleet size, continuous cellular or backhaul connectivity is assumed, and the service is inherently fragile in coverage-constrained regions such as rural corridors, underground passages, and dense urban environments where both GNSS observability and network connectivity are simultaneously degraded.

Beyond the delivery problem, a second and equally significant limitation of existing augmentation systems is their reliance on static, configure-once operational profiles. A traditional RTK broadcast or unicast service is configured with fixed correction update rates, fixed Forward Error Correction (FEC) overhead, fixed modulation and coding schemes, and fixed payload compositions with no mechanism to sense whether field receivers are actually achieving the intended positioning performance. This rigidity results in systematic inefficiency: the service either over-provisions resources to guard against worst-case channel conditions everywhere, or it underperforms in the specific corridors and time windows where the environment is most challenging.

This project addresses both of these structural limitations through the design and software implementation of an Intent-Driven AI-Native Precise Positioning-as-a-Service (PPaaS) platform. The system delivers RTK-class GNSS corrections and bitmap-based positioning assistance over an emulated ATSC 3.0 broadcast transport, and wraps the entire delivery pipeline within a closed-loop AI feedback controller that continuously adapts broadcast parameters in response to observed receiver-population telemetry and high-level operator intents.

1.2 MOTIVATION

The motivation for this work arises at the convergence of three independently mature but historically siloed domains: high-precision GNSS positioning, next-generation digital broadcast infrastructure, and AI-native network management.

High-precision positioning at scale is an emerging operational requirement. Fleet operators of autonomous ground and aerial vehicles cannot economically equip each platform with a dedicated cellular backhaul connection solely for correction delivery, nor can they assume continuous connectivity in rural fringe zones, tunnel corridors, or signal-attenuated urban environments. A one-to-many broadcast delivery mechanism that decouples correction availability from per-vehicle connectivity represents a natural architectural solution one that mirrors how navigation signals themselves are disseminated.

ATSC 3.0, formally standardised through the Advanced Television Systems Committee document suite (A/300, A/321, A/322, A/330, A/331), is the next-generation terrestrial digital television standard deployed across North America and increasingly evaluated internationally. Unlike its predecessors, ATSC 3.0 is built on an IP-native foundation, supports configurable Physical Layer Pipes (PLPs) with independently tunable modulation, coding rate, and robustness, and achieves broadcast ranges of thirty to fifty kilometres per transmitter at data capacities ranging from one to fifty-seven Megabits per second depending on the selected robustness configuration. Critically, the standard supports non-video datacasting through the ROUTE/DASH delivery protocol defined in A/331, which makes it technically straightforward to carry arbitrary IP data payloads including RTCM correction streams and bitmap assistance tiles alongside conventional broadcast content [12]. Prior demonstration work, including a live RTK correction broadcast conducted by MBC Korea over ATSC 3.0 infrastructure, has validated the fundamental feasibility of this transport paradigm. The present project builds on that foundation but introduces a control layer that did not exist in prior demonstrations.

AI-native network management is an architectural paradigm, formalised in standards activities including the ITU-T Focus Group on AI-Native Networks (FG-AINN), where intelligence is embedded into the operational fabric of a service rather than added as a post-hoc optimisation tool. In this paradigm, a service continuously observes outcomes, infers optimal configurations, actuates changes, and validates results in a closed loop. Applied to positioning services, this means that the broadcast delivery strategy FEC overhead, correction update rate, redundancy factor, bitmap tile resolution, PLP robustness is not a set of fixed parameters but a set of policy variables that the AI controller adjusts based on what the vehicle population in the target corridor is actually experiencing. [20] This project is, to the author's knowledge, one of the first implementations to operationalise intent-driven AI-native control specifically for broadcast-delivered GNSS correction services.

1.3 PROBLEM STATEMENT

The specific engineering problem addressed by this project is defined as follows:

Standard GNSS positioning is insufficient for safety-critical autonomous mobility applications, delivering only five-to-ten metre accuracy and collapsing entirely in tunnels, dense urban canyons, and signal-blocked zones. RTK positioning provides centimetre-class accuracy but requires continuous, low-latency delivery of correction messages a requirement that unicast NTRIP-based delivery satisfies only imperfectly due to connectivity constraints, per-receiver bandwidth scaling, and the absence of any feedback on whether corrections are actually enabling receivers to achieve RTK FIX states. [7]

Broadcast-based correction delivery over ATSC 3.0 removes the per-vehicle connectivity dependency and scales naturally to large fleets. However, static broadcast configurations fixed FEC rate, fixed correction rate, fixed modulation and PLP parameters are structurally unable to adapt to the spatial and temporal variability of the positioning environment. A system that broadcasts aggressively everywhere wastes spectrum; a system that broadcasts conservatively fails in challenging corridors.

The problem is therefore to design and implement a software-defined, closed-loop system that:

(a) generates standards-compliant RTCM corrections and bitmap assistance tiles from a GNSS reference segment

(b) delivers them through an ATSC 3.0-compliant broadcast pipeline with configurable ALP encapsulation, LDPC FEC, and OFDM modulation

(c) interprets high-level operator intents expressed in natural language

(d) uses an AI decision engine with uncertainty quantification to continuously adapt broadcast parameters so that positioning service quality measured in RTK FIX availability, horizontal position error, convergence time, and spectral efficiency is optimised against the stated intent.

1.4 OBJECTIVES

The specific objectives pursued in this project are:

- To implement a complete GNSS correction generation pipeline that produces RTCM 3.x binary frames (Messages 1005, 1077, 1087) from either real RINEX observation files or deterministic synthetic scenario data, with intelligent coordinate resolution from RINEX headers, scenario profiles, or CLI defaults.

- To construct an ATSC 3.0-compliant broadcast processing chain comprising an ALP encoder (per A/330), a multi-mode OFDM modulator with configurable FFT sizes (8K, 16K, 32K), guard intervals, and modulation orders (QPSK through 256-QAM), and a multi-PLP broadcast scheduler and controller.

- To design and train a neural network-based AI decision model that ingests fifty-dimensional telemetry and intent feature vectors and outputs five broadcast configuration parameters redundancy ratio, spectrum allocation, availability target, convergence time, and accuracy HPE using residual network architecture with Monte Carlo Dropout for uncertainty quantification.

- To implement a natural-language intent parser capable of classifying operator intents into three canonical categories Maximize Accuracy, Maximize Reliability, and Optimize Spectrum using a pretrained sentence transformer backbone augmented by a learned thirty-two-dimensional intent embedding network.

- To define and execute three canonical test scenarios open-sky rural (Scenario 1), suburban sparse RTCM (Scenario 2), and urban canyon multipath (Scenario 3) and to characterise the behaviour of the traditional static broadcast baseline against an AI-adaptive policy across these scenarios.

- To evaluate system performance using a rigorous metrics framework computing horizontal position error (RMS and 95th percentile), RTK FIX availability, mode transition counts, time-to-first-fix, spectral efficiency, and data recovery rates.

1.5 SCOPE AND LIMITATIONS

The scope of this project is a fully software-defined, simulation-based implementation. No over-the-air ATSC 3.0 RF transmission is performed, and no hardware GNSS receivers or ATSC 3.0 demodulators are used in the evaluation. The broadcast pipeline emulates ATSC 3.0 signal processing in software using standard open-source numerical libraries, and the positioning pipeline is driven either by real RINEX observation data processed through RTKLIB or by deterministic synthetic traces generated by the scenario simulator.

Sub-centimetre or sub-three-centimetre positioning accuracy was not achieved within this implementation phase. The primary measured accuracy figures reflect the traditional static baseline, which in the urban canyon scenario produced RTK FIX in only 6.7% of epochs. The AI controller demonstrated measurable improvement in reliability and data recovery a 45.6% increase in recovered bytes in the urban canyon reliability scenario but full centimetre-class accuracy validation against real field data remains as future work. Additionally, the Reed-Solomon outer FEC component was implemented using the reedsolo library but was not rigorously validated end-to-end; complete outer code integration and bit-error-rate characterisation constitute a separate future work item.

CHAPTER-2

LITERATURE REVIEW

2.1 INTRODUCTION

The problem of delivering high-precision GNSS corrections reliably and adaptively to large populations of mobile receivers sits at the intersection of several active research and standardisation streams. This chapter surveys the relevant literature across five domains: standard and high-precision GNSS positioning techniques, correction delivery mechanisms and their limitations, ATSC 3.0 broadcast infrastructure and its application to non-video datacasting, machine learning approaches to GNSS performance enhancement, and intent-driven AI-native network management paradigms. For each domain, key contributions are reviewed, and the gap that the present project specifically addresses is identified.

2.2 STANDARD GNSS AND RTK POSITIONING

The Global Positioning System Standard Positioning Service (GPS SPS) performance specification, maintained by the U.S. Space Force, characterises baseline GPS horizontal accuracy at approximately five to ten metres under open-sky conditions for a single-frequency, code-phase receiver. Multi-constellation receivers incorporating signals from GLONASS, Galileo, and BeiDou improve availability and dilution of precision (DOP) figures, particularly in environments with reduced sky visibility, but do not fundamentally alter the metre-class accuracy ceiling imposed by code-phase measurements. [1]

The Real-Time Kinematic technique overcomes this ceiling by exploiting carrier-phase observables, whose wavelength of approximately nineteen to twenty-four centimetres for L1 GPS provides a measurement precision far exceeding code-phase. RTK resolves integer carrier-phase ambiguities using relative positioning between a rover receiver and a reference station with known coordinates. Takasu and Yasuda, in their foundational work presenting the open-source RTKLIB package, demonstrated that carrier-phase ambiguity resolution with centimetre-class accuracy is achievable using commodity hardware when high-quality corrections are continuously available. RTKLIB supports RTCM v3.x correction messages, multi-constellation processing, and several ambiguity resolution algorithms, and has become the de facto reference implementation for RTK research and this project's positioning engine [21].

2.3 RTCM CORRECTION FORMATS AND INTERPORTABILITY

Correction message interoperability across reference infrastructure, network providers, and heterogeneous receivers is principally achieved through the RTCM v3.x standard, maintained by the Radio Technical Commission for Maritime Services [4]. The message set includes reference station coordinate messages (Type 1005), multi-signal measurement messages (MSM Types 1074–1127) covering GPS, GLONASS, Galileo, and BeiDou constellations, and network correction messages for wider-area applications. The MSM7 family (Types 1077, 1087, 1097, 1127) provides full-resolution pseudorange, carrier-phase, Doppler, and signal-to-noise data and represents the current best practice for interoperable RTK correction delivery [4].

Septentrio and NovAtel receiver documentation confirms that MSM-based RTCM streams are universally supported across professional and mass-market GNSS chipsets, ensuring that a broadcast delivery system carrying standard RTCM payloads does not require any modification to the receiver-side positioning stack a design principle explicitly preserved in the present project, where RTCM frames generated by the correction pipeline remain semantically unchanged regardless of the broadcast transport or AI policy applied [5].

2.4 CORRECTION DELIVERY MECHANISMS: NTRIP AND ITS LIMITATIONS

The dominant deployed mechanism for RTK correction delivery is NTRIP, a protocol developed jointly by the Federal Agency for Cartography and Geodesy (BKG) and the European Space Agency (ESA), which streams RTCM data over HTTP to individual receivers. NTRIP is technically effective for low-to-moderate fleet densities in areas with reliable cellular or broadband connectivity but introduces four well-understood operational constraint.

First, bandwidth demand scales linearly with the number of connected receivers, as each receiver maintains an independent TCP session with the correction server. Second, the protocol inherits all connectivity dependencies of the underlying cellular or IP network, making it fragile in rural fringe zones, underground structures, and dense urban corridors where both GNSS and cellular performance degrade simultaneously. Third, NTRIP provides no mechanism for the correction server to observe whether individual receivers are achieving RTK FIX the delivery is entirely open-loop. Fourth, the protocol is fundamentally unicast, making it unsuitable for scenarios where thousands of vehicles must simultaneously receive identical correction data across a wide geographic area[7] [8].

These limitations are not merely theoretical. Field studies of NTRIP-based RTK systems in rural agricultural corridors and urban test beds consistently report intermittent correction availability as the primary cause of RTK performance degradation, particularly during periods of cellular network congestion or partial connectivity outages. The present project specifically targets these gaps by replacing per-receiver unicast sessions with a single broadcast transmission reaching all receivers in the coverage area simultaneously.

2.5 ATSC 3.0 AS A BROADCAST DELIVERY PLATFORM

ATSC 3.0, formally specified in the A/300 system standard, is the third generation of terrestrial digital television developed by the Advanced Television Systems Committee [9]. Unlike its predecessors, it is built on a fully IP-native architecture and introduces a layered design in which the physical, link, and application layers are independently configurable a flexibility that makes it directly applicable to non-video datacasting applications including positioning correction delivery.

The physical layer, specified in A/322, implements an OFDM-based waveform with configurable Fast Fourier Transform sizes (2K through 32K subcarriers), guard intervals ranging from 1/128 to 1/4 of the symbol period, and a modulation order range from QPSK to 4096-QAM [10]. Forward error correction is implemented as a concatenated scheme combining Low-Density Parity-Check (LDPC) codes at rates from 2/15 to 13/15 with a Bose–Chaudhuri–Hocquenghem (BCH) outer code. The system supports Physical Layer Pipes (PLPs), which are independently configurable logical channels within a single broadcast multiplex allowing, for example, a highly robust low-rate PLP for mission-critical corrections alongside a higher-rate PLP for assistance maps and integrity data, as exploited in the present project.

The link layer is specified in A/330 through the ATSC Link-Layer Protocol (ALP), which encapsulates IPv4, IPv6, and compressed IP packets into variable-length ALP packets with headers carrying packet type, length, and timing information [11]. The signalling, delivery, synchronisation, and error protection layer is specified in A/331, which defines the ROUTE/DASH delivery protocol for file and streaming data, the Service Layer Table (SLT) and Low-Level Signalling (LLS) descriptors used for service discovery, and the application-layer FEC framework. Together, A/330 and A/331 define the complete path from IP payload to broadcast receiver, and both are implemented at a software simulation level in the broadcast pipeline of the present project [12].

ATSC 3.0 provides broadcast coverage of thirty to fifty kilometres per transmitter at data capacities ranging from approximately one to fifty-seven Megabits per second depending on the selected robustness configuration, and supports mobile reception at speeds exceeding two hundred kilometres per hour [9] [10]. The combination of wide coverage, scalable capacity, and spectrum already licensed for terrestrial broadcast makes it a compelling delivery vehicle for positioning correction services that must reach large, mobile populations without per-receiver infrastructure cost.

The practical feasibility of carrying RTK corrections over ATSC 3.0 was demonstrated by Shin et al. at the 2021 IEEE Broadband Multimedia Systems and Broadcasting symposium, where MBC Korea transmitted RTCM correction data alongside standard television content over a live ATSC 3.0 multiplex [13]. Receiving vehicles equipped with ATSC 3.0 demodulators successfully decoded RTK corrections and achieved centimetre-level positioning. This demonstration establishes the technical viability of the transport and provides a direct precedent for the present project. However, the MBC demonstration employed a static broadcast configuration with no adaptive control or feedback loop precisely the gap that this project targets.

The NAB Broadcast Positioning System initiative further demonstrates institutional recognition of ATSC 3.0 as a positioning infrastructure, exploiting the precise timing available in ATSC 3.0 bootstrap signals for time-of-arrival ranging in addition to correction delivery [14]. The GPS.gov ATSC 3.0 positioning evaluation conducted at CGSIC corroborates that broadcast signals provide useful timing and positioning augmentation beyond conventional correction streaming [15]. These parallel developments reinforce the relevance of the chosen transport for the present project.

2.6 MACHINE LEARNING FOR GNSS ENHANCEMENT

The application of machine learning to GNSS positioning has been investigated primarily at the receiver side, targeting two related problems: non-line-of-sight (NLOS) signal detection and multipath error mitigation. Crespillo et al. proposed a robust ML-based NLOS detection framework using receiver signal features, demonstrating that classifier-based identification of corrupted measurements can significantly reduce the frequency and magnitude of large, non-Gaussian positioning errors in urban environments. Their approach, evaluated across multiple urban test scenarios, showed that selective exclusion of NLOS-affected measurements substantially improves RTK availability in dense city blocks the same operational environment that constitutes Scenario 3 in the present project.

Xu et al. conducted a comprehensive review and benchmark of machine learning approaches to GNSS multipath and NLOS mitigation, comparing deep learning classifiers, support vector machines, and random forest methods across multiple datasets [17]. Their analysis identifies receiver-observable features carrier-to-noise density, elevation angle, pseudorange residuals, and code-carrier divergence as the most informative inputs for NLOS classifiers, and concludes that deep models consistently outperform shallow classifiers when training data is sufficient [17]. Critically, both studies restrict their scope to receiver-side intelligence and assume that corrections are reliably available from the delivery channel they do not address the problem of adapting the delivery channel itself based on observed positioning outcomes.

This gap in the receiver-side ML literature optimising the estimator while leaving the delivery mechanism static motivates the present project's architectural choice to locate the AI controller at the service orchestration layer rather than at the receiver. The project does not modify the receiver-side RTK engine but instead adapts the broadcast policy through which corrections and assistance are delivered, using population-level positioning telemetry as the feedback signal.

2.7 INTENT DRIVEN AI-NATIVE NETWORK MANAGEMENT

The concept of intent-driven network management originates in the autonomic networking research community and has been substantially advanced through 3GPP, IETF, and ITU-T standardisation activities over the past decade. Mehmood et al. conducted a structured literature review of intent-driven autonomous network management in future cellular networks, identifying closed-loop control, intent translation, and outcome validation as the three essential architectural components of a self-managing network. Their taxonomy distinguishes between reactive systems which respond to observed violations and proactive systems which anticipate degradation using predictive models and concludes that production-grade intent-driven management requires both.

Banerjee, Mwanje, and Carle presented a specific architecture for intent-driven orchestration of cognitive autonomous networks in the Radio Access Network (RAN) management domain, demonstrating that high-level service intents such as "guarantee five-nines availability in corridor X" can be automatically translated into concrete RAN configuration policies through a combination of intent parsing, policy templates, and a feedback loop comparing observed KPIs against intent thresholds [19]. Their architectural decomposition intent interface, policy engine, actuation layer, and telemetry feedback is directly mirrored in the AI subsystem of the present project, adapted from the cellular RAN domain to broadcast-delivered positioning services.

The ITU-T Focus Group on AI-Native Networks (FG-AINN), active since 2023, has formalised the concept of AI-nativeness as a design property of future network functions, distinguishing AI-native systems where intelligence is embedded from inception and continuously self-validates from AI-assisted systems where ML models are retrofitted to existing architectures [20]. The present project is explicitly designed as AI-native in the FG-AINN sense: the AI controller is not an optional add-on to the broadcast pipeline but is the primary decision-making entity that determines all broadcast configuration parameters, with a rule-based fallback activated only when model confidence falls below a defined threshold.

2.8 IDENTIFIED GAPS IN THE LITERATURE

A synthesis of the reviewed literature reveals four specific gaps that the present project addresses and that have not been collectively resolved by any prior work:

Gap 1 — No closed-loop outcome-driven broadcast adaptation: Prior work on ATSC 3.0 correction delivery, including the MBC demonstration and the NAB BPS initiative, establishes delivery feasibility but operates with static broadcast configurations. No prior system continuously measures receiver-population positioning outcomes and uses them to adapt broadcast policy in real time.

Gap 2 — Receiver-side ML does not address the delivery channel: Machine learning work on NLOS detection and multipath mitigation improves the estimator but assumes corrections are reliably available. No prior work applies ML at the broadcast service orchestration layer to adapt delivery robustness based on observed field performance.

Gap 3 — Intent-driven management has not been applied to positioning services: The intent-driven orchestration architectures of Mehmood et al. and Banerjee et al. operate in the cellular RAN domain. The translation of this paradigm to broadcast-delivered positioning services where the control variables are PLP robustness, correction update rate, redundancy, and tile resolution rather than handover thresholds and power levels is novel.

Gap 4 — No prior system integrates all five layers end-to-end in software: Existing work addresses individual components the RTK engine, the broadcast transport, or the ML estimator in isolation. The present project integrates GNSS correction generation, ATSC 3.0 broadcast processing, receiver-side RTK evaluation, and AI-native closed-loop control into a single coherent software-defined system.

CHAPTER 3

SYSTEM ARCHITECTURE AND DESIGN

3.1 INTRODUCTION

This chapter presents the complete architectural design of the Intent-Driven AI-Native Precise Positioning-as-a-Service (PPaaS) platform. The system is conceived as a five-layer stack in which each layer has a clearly defined responsibility, a well-specified interface with its neighbours, and a set of configurable parameters that are exposed as controllable knobs to the AI orchestrator. The design philosophy prioritises three properties above all others: standards compliance at the correction and broadcast layers, so that the receiver-side positioning stack requires no modification; adaptability at the orchestration layer, so that broadcast policy can be updated in real time without disrupting ongoing correction delivery; and observability throughout the pipeline, so that telemetry from field receivers provides a continuous, quantitative basis for control decisions.

The overall platform integrates five logical subsystems: a GNSS reference and correction generation segment, an AI-native positioning orchestrator that functions as the central decision-making entity, an ATSC 3.0 broadcast processing chain that translates orchestrator commands into physical-layer transmissions, a receiver-side pipeline that consumes broadcast corrections and produces positioning solutions, and a telemetry and evaluation framework that closes the feedback loop by computing KPIs and returning them to the orchestrator.

3.2 TOP-LEVEL SYSTEM OVERVIEW

The five layers of the system are illustrated in Figure 3.1 and described in detail in the subsections that follow.

Figure 3.1: Top-level architecture of the Intent-Driven AI-Native PPaaS platform showing the five logical layers and data flow from GNSS reference station to vehicle receiver and back through the AI feedback loop.

Layer 1 — GNSS Reference and Correction Generation Segment: A multi-constellation GNSS receiver occupying a surveyed position of millimetre-class accuracy continuously processes raw satellite observations to compute per-satellite error vectors, atmospheric delay estimates, and clock offsets. These measurements are encoded into RTCM 3.x binary correction frames at a configurable update rate and supplemented by bitmap-based coverage quality tiles representing the spatial distribution of positioning reliability across the service area.

Layer 2 — Broadcast Core (AI-Native Positioning Orchestrator): The orchestrator is the intelligence centre of the platform. It receives aggregated telemetry from field receivers, interprets operator-supplied service intents, runs neural-network inference to determine the optimal broadcast configuration, validates model confidence, and emits configuration commands to the broadcast processing chain. When model confidence falls below a defined threshold, a rule-based fallback policy is applied to ensure service continuity.

Layer 3 — ATSC 3.0 Broadcast Processing Chain: This layer receives correction data and configuration commands from the orchestrator and executes the physical delivery. It encapsulates RTCM frames and bitmap tiles into ALP packets per ATSC A/330, applies FEC protection per A/322, modulates the resulting bitstream onto an OFDM waveform, and schedules transmission across one or more Physical Layer Pipes (PLPs) with independently configured robustness parameters.

Layer 4 — Receiver and RTK Positioning Stack: At the vehicle or drone, an ATSC 3.0 client demodulates the broadcast signal, extracts ALP packets, and delivers RTCM corrections and bitmap tiles to the RTK positioning engine (RTKLIB). The engine fuses broadcast corrections with local GNSS measurements to compute a carrier-phase position solution, generating a solution quality indicator (FIX / FLOAT / SINGLE) and position residuals at each epoch.

Layer 5 — Telemetry and Evaluation Framework: Positioning outcomes, solution quality flags, correction age indicators, and channel quality metrics are aggregated across the receiver population in the target corridor and returned to the orchestrator, closing the feedback loop. The evaluation framework additionally computes scenario-level KPIs HPE_RMS, HPE_p95, FIX availability, convergence time, and spectral efficiency for performance reporting.

3.3 CLOSED LOOP CONTROL ACHITECTURE

Figure 3.2: Sequence diagram of the closed-loop broadcast-assisted RTK pipeline. The base station generates RTCM corrections that are packaged, FEC-protected, and broadcast over ATSC 3.0. Vehicles apply corrections in the RTK engine and report aggregated telemetry back to the AI controller, which updates the broadcast configuration to maintain positioning performance.

Stage 1 — Observe: Field receivers report structured telemetry containing RTK solution mode, position error estimates, satellite metrics, channel quality indicators, and environment state flags at each epoch.

Stage 2 — Decide: The AI decision engine constructs a fifty-dimensional feature vector from the aggregated telemetry, the encoded intent embedding, and current broadcast channel state. This vector is passed to the trained neural network, which runs forward inference with Monte Carlo Dropout enabled across twenty stochastic passes. The mean of the twenty predictions forms the nominal decision; the standard deviation across passes provides an uncertainty estimate. If the derived confidence score defined as one minus the normalised uncertainty exceeds 0.85, the neural network output is accepted. If confidence is below this threshold, a rule-based fallback policy is selected from three pre-defined profiles (Conservative, Balanced, Aggressive) based on recent confidence history and the current uncertainty magnitude.

Stage 3 — Act: The accepted decision is translated into a concrete broadcast configuration command an AIBroadcastCommand object specifying FEC LDPC code rate, FEC overhead percentage, modulation scheme, FFT size, guard interval, correction update frequency, tile resolution, PLP mode, and redundancy factor. This command is validated by the CommandValidator before being applied to the broadcast controller. The broadcast controller updates the transmission chain parameters in the next scheduling cycle.

Stage 4 — Validate: After a defined observation window, the telemetry aggregator recomputes KPIs for the updated configuration and compares them against the intent's constraint thresholds (target HPE, minimum FIX availability, maximum spectrum budget, maximum convergence time). If the constraints are satisfied, the policy is held; if not, the control cycle repeats with a new inference pass incorporating the latest telemetry, triggering further adaptation.

3.4 GNSS LAYER DESIGN

3.4.1 RTCM CORRECTION GENERATOR

The correction generator produces RTCM 3.x binary frames from either real RINEX observation files processed through RTKLIB's convbin utility or a deterministic synthetic generator for scenarios where real data is unavailable. The frame format is illustrated in Figure 3.3.

Figure 3.3: RTCM 3.x binary frame structure. The preamble byte (0xD3) marks frame start, followed by a six-bit reserved field and a ten-bit payload length, a twelve-bit message type identifier, the variable-length payload, and a three-byte CRC-24Q for error detection.

The generator produces three message types in the present implementation:

- Message 1005 (Stationary RTK Reference Station ARP): Carries the antenna reference point coordinates in ECEF (Earth-Centred Earth-Fixed) frame to 0.0001 metre resolution. Transmitted once at stream initialisation [4].

- Message 1077 (GPS MSM7): Full-resolution GPS carrier-phase, pseudorange, Doppler, and carrier-to-noise density for all tracked satellites. Transmitted at the configured correction update rate [5].

- Message 1087 (GLONASS MSM7): Equivalent MSM7 message for GLONASS constellation observables [6].

A three-tier coordinate resolution chain ensures robustness across deployment contexts: the generator first attempts to extract station coordinates from the RINEX observation file header (APPROX POSITION XYZ field), falls back to a scenario profile JSON if RINEX extraction fails, and finally uses CLI-provided defaults. CRC-24Q integrity checking is applied to all generated frames using the standard RTCM polynomial (0x1864CFB).

3.4.2 COVERAGE MAP AND BITMAP TILE GENERATOR

Bitmap tiles provide a spatial representation of positioning quality across the service area, encoding three coverage states: reliable correction coverage (white), blocked or obstructed zones (black), and degraded multipath-prone areas (grey). Tiles are generated at configurable resolutions (100×100 to 200×200 pixels) representing geographic grid cells. Separate tile sets are produced for rural corridor, urban canyon block, and drone flight corridor scenarios. Tile resolution and update rate are exposed as AI-controllable parameters under the Maximize Reliability intent, the AI increases tile resolution and refresh frequency for urban multipath zones; under the Optimize Spectrum intent, lower-resolution tiles with reduced refresh rate are selected.

3.4.3 SCENARIO SIMULATOR

For Scenarios 2 and 3, where real GNSS field data is unavailable, the scenario simulator generates deterministic synthetic datasets seeded for reproducibility. The simulator produces:

- Ground truth trajectory (gt.csv): Vehicle position sequence with smooth heading changes, speed variations, and height evolution using a seeded random walk in the Seoul area coordinate frame.

- RTCM corrections schedule (corrections_schedule.json, Scenario 2): Sparse correction availability modelling five-to-ten second update intervals with a fifteen percent random dropout probability per interval, representing low-bandwidth rural coverage conditions.

- Multipath profile (multipath_profile.json, Scenario 3): Per-epoch pseudorange bias (0.5–5.0 m), noise standard deviation, NLOS probability (spatially correlated, mean 40%), sky-view percentage, and visible satellite count modelling urban canyon conditions.

- Mode transitions (mode_transitions.csv, Scenario 3): Per-epoch RTK mode (FIX/FLOAT/SINGLE) derived from multipath severity and satellite visibility, with hysteresis to model the temporal persistence of mode states.

3.5 DATA FORMATS AND INTERFACE SPECIFICATIONS

The system uses three primary structured data formats for inter-module communication, shown in Figure 3.4.

Figure 3.4:Bottom — Vehicle telemetry JSON record containing RTK solution metrics, environment flags, and vehicle state. Top — AI broadcast command JSON showing the output of the orchestrator including intent, broadcast configuration parameters, reasoning, and confidence score.

The Vehicle Telemetry JSON record is the primary feedback signal from field receivers to the orchestrator. It captures: UNIX timestamp, vehicle identifier, ECEF or geodetic position, RTK mode, position error estimate, number of satellites used, signal strength, convergence time, environment state flags (urban density scalar, tunnel indicator, canyon indicator, blockage prediction), vehicle speed, heading, and an overall solution confidence score.

The AI Broadcast Command JSON record is the primary actuation signal from the orchestrator to the broadcast controller. It carries: command identifier, timestamp, intent name, broadcast configuration block (FEC LDPC code rate, FEC overhead percentage, modulation scheme, FFT size, guard interval, correction update frequency, tile resolution, PLP mode, redundancy factor), constraint block, reasoning text, and model confidence score.

The RTCM binary stream carries correction frames in the standard RTCM 3.x format described in Section 3.4.1, with no proprietary modifications. This ensures that any standards-compliant RTK receiver can consume the corrections without modification to the receiver-side software stack.

3.6 BROADCAST PROCESSING LAYER DESIGN

3.6.1 ATSC 3.0 STANDARDS ALIGNMENT

The broadcast processing chain is designed to comply with the ATSC 3.0 standard suite. Table 3.1 maps each implemented module to its governing standard document.

| Module | Standard | Key Parameters |
| --- | --- | --- |
| ALP Encoder | A/330 | Packet type, length, header flags, fragmentation |
| FEC Encoder (LDPC) | A/322 | Code rate 2/15–13/15, BCH outer code |
| OFDM Modulator | A/322 | FFT: 8K/16K/32K, GI: 1/4–1/32, Mod: QPSK–256-QAM |
| Frame Builder | A/331 | SLT/LLS service signalling, ROUTE/DASH packaging |
| PLP System | A/322 | PLP-A (corrections), PLP-B (tiles/integrity), PLP-C (optional) |
| Broadcast Scheduler | A/331 | Priority scheduling, redundancy insertion |
| Broadcast Controller | A/300 | Intent-to-config translation, mode management |

3.6.2 PHYSICAL LAYER PIPE ARCHITECTURE

The system allocates three Physical Layer Pipes for positioning services:

- PLP-A (RTK Correction Stream): Carries RTCM correction frames. Configured for high robustness under the Maximize Reliability intent (low-order modulation, low LDPC code rate, maximum FEC overhead). Under the Optimize Spectrum intent, modulation order is increased and FEC overhead reduced to improve spectral efficiency.

- PLP-B (Bitmap Tiles and Integrity): Carries coverage quality tiles and integrity alert messages. Tile resolution and update frequency are AI-controlled parameters.

- PLP-C (Optional Extended Services): Reserved for experimental services including extended coverage or lower-priority assistance data.

3.6.3 BROADCAST CONTROLLER OPERATING MODES

The broadcast controller supports three operating modes that cover the full range of deployment scenarios evaluated in this project:

Traditional Mode: All broadcast parameters are fixed at a static baseline profile. The FEC code rate is set to RATE_8/15, modulation to QPSK, FFT size to 8K, guard interval to 1/8. This mode serves as the V4 baseline in the technology variant comparison. No telemetry is consumed and no adaptation occurs.

AI-Controlled Mode: The controller accepts configuration commands from the AI orchestrator and applies them to all downstream processing stages. This is the V5 variant in the evaluation. The CommandValidator enforces parameter range constraints before any command is applied.

3.7 AI Orchestration Layer Design

3.7.1 INTENT PARSER ARCHITECTURE

The intent parser translates free-text operator inputs into structured CanonicalIntent objects. It uses a two-stage architecture: a pretrained sentence transformer (all-MiniLM-L6-v2) produces a 384-dimensional semantic embedding of the input text, which is passed through a learned 384→256→128→32 dimensional projection network to produce a 32-dimensional intent embedding. Simultaneously, a keyword and phrase matching engine computes confidence scores for each of the three canonical intent classes. The detected intent type, validated constraint parameters, and intent embedding are packaged into a Canonical-Intent record that is consumed by the decision model.

Table 3.2: The three canonical intents and their default constraint thresholds are:

| Intent | HPE Target | Min FIX Availability | Max Spectrum | Max Convergence |
| --- | --- | --- | --- | --- |
| Maximize Accuracy | 3.0 cm | 95% | 2.0 Mbps | 30s |
| Maximize Reliability | 10.0 cm | 98% | 3.0 Mbps | 45s |
| Optimize Spectrum | 15.0 cm | 90% | 1.5 Mbps | 60s |

3.7.2 DECISION MODEL ARCHITECTURE

The neural network decision model maps the fifty-dimensional input feature vector to five broadcast configuration output parameters. The architecture employs a residual network design with the following layer sequence: a 50→128 input projection with BatchNorm and Dropout(0.3), two 128-dimensional residual blocks (each consisting of two linear layers with BatchNorm and skip connections), a 128→64 transition layer, a 64→32 compression layer, and five independent single-neuron output heads followed by sigmoid activation. The output heads predict: redundancy ratio, spectrum allocation in Mbps, RTK FIX availability fraction, convergence time in seconds, and accuracy as HPE in centimetres.

Monte Carlo Dropout is applied during inference by retaining dropout active across twenty forward passes. The variance across passes provides a per-output uncertainty estimate; the overall model confidence is computed as one minus the normalised mean variance. Decisions with confidence below 0.85 are routed to the rule-based fallback.

3.7.3 FALLBACK POLICY DESIGN

Three fallback policy profiles are pre-defined for use when model confidence is insufficient:

- Conservative: High redundancy (3.5×), low spectrum (1.2 Mbps), 95% availability target. Applied when adjusted confidence < 0.4.

- Balanced: Moderate redundancy (2.5×), moderate spectrum (1.5 Mbps), 90% availability. Applied when adjusted confidence is between 0.4 and 0.7.

- Aggressive: Lower redundancy (1.8×), higher spectrum (1.8 Mbps), 87% availability. Applied when confidence > 0.7 but uncertainty exceeds threshold.

The selection among fallback policies is determined by a weighted combination of recent confidence history from a sliding window and the current uncertainty estimate from the MC Dropout inference pass.

3.8 SOFTWARE ARCHITECTURE AND REPOSITORY STRUCTURE

The complete system is implemented in Python 3.9+ and organised into three primary module groups within the project repository, shown in Figure 3.6.

Figure 3.5: Project repository directory structure showing the three primary module groups (gnss/, broadcast/, ai/), the integration layer and documentation.

The three module groups correspond directly to the three functional subsystems:

- gnss/: Contains rtcm_generator_v2.py, coverage_map_generator.py, scenario_simulator.py, and rtk_evaluate.py. Responsible for correction generation, scenario data synthesis, and positioning evaluation.

- broadcast/: Contains alp_encoder.py, fec_encoder.py, ofdm_modulator.py, frame_builder.py, decoder.py, channel_simulator.py, pipeline.py, broadcast_controller.py, and rtcm_to_broadcast_handoff_v2.py. Responsible for the complete ATSC 3.0 processing chain.

- ai/: Contains ai_intent_parser.py, ai_broadcast_decision_model_v2.py, ai_data_preprocessor.py, ai_inference_engine_v2.py, and ai_feedback_loop.py. Responsible for intent interpretation, decision making, and closed-loop control.

Inter-module communication is structured through typed dataclass interfaces, CanonicalIntent, AIBroadcastCommand, BroadcastDecision, InferenceResult, ControllerMetrics, ensuring that each subsystem can be independently tested and replaced without cascading interface changes.

CHAPTER 4

IMPLEMENTATION

4.1 INTRODUCTION

This chapter describes the concrete software implementation of each module across the three subsystems of the PPaaS platform the GNSS correction generation pipeline, the ATSC 3.0 broadcast processing chain, and the AI orchestration layer. For each module, the design decisions, key algorithms, data structures, and interface contracts are documented in sufficient technical depth to allow reproduction. Code structure is described with reference to specific classes and functions in the implementation files. The chapter concludes with a description of the end-to-end integration pipeline that connects all three subsystems into a single executable workflow [21].

All modules are implemented in Python 3.9 or above using exclusively open-source libraries. No commercial software or proprietary toolchains are required. The complete implementation uses RTKLIB for RTK positioning, NumPy and SciPy for numerical signal processing, PyTorch for neural network training and inference, the HuggingFace Transformers library for the pretrained sentence encoder, and Pandas and Matplotlib for data analysis and visualisation.

4.2 GNSS CORRECTION GENERATION SUBSYSTEM

4.2.1 RTCM GENERATOR

The RTCM generator is the entry point of the correction pipeline. Its primary responsibility is to produce a standards-compliant binary RTCM 3.x stream that can be consumed without modification by any RTK positioning engine, including RTKLIB.

Frame Construction. Every RTCM 3.x frame follows a fixed structural layout: a one-byte preamble (0xD3), a two-byte header encoding six reserved bits and a ten-bit payload length, a variable-length payload whose first twelve bits carry the message type identifier, and a three-byte CRC-24Q integrity checksum. The CRC computation uses the standard RTCM polynomial 0x1864CFB applied over the concatenated header and payload bytes. The build_rtcm_frame() function encapsulates this construction and is called by all message-specific generators.

Message 1005 — Station ARP. The station ARP message carries the antenna reference point coordinates as 38-bit signed integers representing ECEF X, Y, Z in units of 0.0001 metres. The generate_msg1005() function encodes the resolved station coordinates and packs the message type, station identifier, and coordinate fields into the payload bytearray before invoking frame construction [4].

Message 1077 — GPS MSM7. The GPS MSM7 message carries full-resolution GPS observables — pseudorange, carrier phase, Doppler, and carrier-to-noise density — for all tracked satellites. The present implementation generates a deterministic stub MSM7 for pipeline testing purposes, populating satellite mask, signal mask, and per-satellite data fields with parameterised values. The stub is epoch-stamped using GPS time-of-week in milliseconds derived from the epoch counter and configured interval [5].

Message 1087 — GLONASS MSM7. Generated using the same stub architecture as Message 1077, adapted for the GLONASS satellite numbering and frequency slot conventions [6].

Coordinate Resolution Chain. A critical design feature of the generator is its three-tier coordinate resolution mechanism, implemented in resolve_station_coordinates(). The function first attempts to extract station coordinates from the RINEX observation file header by parsing the APPROX POSITION XYZ field using fixed-width column parsing with multi-encoding fallback (UTF-8, latin-1, ISO-8859-1). If RINEX extraction fails or no RINEX file is available, it falls back to a scenario profile JSON. If neither source is available, CLI-provided defaults or hardcoded test coordinates are used. This chain ensures the generator produces geographically correct corrections across all deployment modes without requiring manual configuration in common cases.

Operational Modes. The generator supports two operational modes selected at runtime: a real-data mode that invokes RTKLIB's convbin utility to convert a provided RINEX observation and navigation file pair into a standards-compliant RTCM binary stream, and a synthetic mode that uses the internal stub generators to produce a test stream of configurable epoch count and satellite count. A --strict-real flag disables the synthetic fallback for production deployments where correction authenticity is mandatory.

4.2.2 COVERAGE MAP GENERATOR

The coverage map generator produces rasterised bitmap tiles representing the spatial distribution of positioning quality across the service area. Each tile is a two-dimensional NumPy array of configurable resolution (default 100×100 pixels) with pixel values encoding one of three coverage states: 255 (reliable), 128 (degraded/multipath-prone), or 0 (blocked). Tiles are generated independently for each scenario type and geographic region.

For rural corridor scenarios, the tile generator produces a predominantly white (reliable) field with narrow grey bands representing tree lines or terrain obstructions at the corridor edges. For urban canyon scenarios, a structured pattern of black rectangular blocks representing buildings alternates with grey inter-building corridors, reflecting the spatial pattern of signal blockage in a dense city grid. Tile size, block dimensions, and blockage density are parameterised and stored in the scenario profile JSON.

The generator also computes a coverage quality scalar per tile the fraction of reliable pixels which is included in the tile metadata and used by the AI controller as one of the fifty input features for the decision model.

4.2.3 SCENARIO SIMULATOR

The scenario simulator generates complete synthetic datasets for Scenarios 2 and 3. All outputs are deterministic given the same random seed, enabling reproducible evaluation runs. The simulator is invoked with a scenario identifier, seed value, and epoch count.

Ground Truth Trajectory Generation. The generate_ground_truth() function produces a vehicle trajectory as a sequence of GroundTruthEpoch records, each containing epoch index, timestamp in seconds-of-day, geodetic coordinates, velocity, and heading. The trajectory evolves through a seeded random walk in heading (with scenario-specific heading noise standard deviations of 2° for Scenario 2 and 15° for Scenario 3 to model highway and urban driving respectively) and speed variation (±20% around a base speed of 20 m/s for Scenario 2 and 8 m/s for Scenario 3). Coordinate updates are computed using a planar approximation converting metres to degrees via the local scale factors at the Seoul area reference coordinates.

RTCM Corrections Schedule (Scenario 2). The generate_corrections_schedule() function models the sparse correction availability of a low-bandwidth rural ATSC coverage zone. Correction availability is structured as a sequence of intervals of five to ten epochs duration, each with an independent 15% probability of dropout (network outage). Available intervals represent successful correction delivery; dropout intervals represent periods where no valid correction is received by the rover.

Multipath Profile (Scenario 3). The generate_multipath_profile() function models the urban canyon signal environment as a spatially correlated severity process. A severity scalar (0 = open sky, 1 = deep canyon) evolves as a bounded random walk with standard deviation 0.05 per epoch, representing the gradual transition between open intersections and deep canyon segments as the vehicle moves through the urban grid. For each epoch, pseudorange bias is drawn from a scaled uniform distribution over the range 0.5–5.0 metres (scaling with severity), additional noise standard deviation from 0.1–1.0 metres, NLOS probability as a product of base probability (0.4) and severity, sky-view percentage as 100 minus severity scaled to 10–90%, and visible satellite count as 12 minus severity-scaled reduction. These parameters collectively determine the RTK mode assignment in the subsequent mode transition generation step.

Mode Transitions (Scenario 3). The generate_mode_transitions() function translates per-epoch multipath and satellite visibility conditions into RTK mode states using a probabilistic state machine with hysteresis. Under NLOS conditions or fewer than five visible satellites, the system transitions to STANDALONE (mode code 5) with 70% probability. Under moderate conditions (sky view 40–60%, satellites 5–8), FLOAT (mode code 2) is most probable. FIX (mode code 1) is assigned only under favourable conditions. A 30% persistence factor prevents mode oscillation at every epoch, modelling the temporal correlation of real RTK mode behaviour.

4.2.4 RTK EVALUATION ENGINE

The evaluation engine is the measurement layer of the system. It orchestrates RTKLIB baseline and optimised positioning runs for each scenario, parses the resulting .pos solution files, computes the full KPI set, and writes structured comparison outputs.

RTKLIB Invocation. For Scenarios 1 and 2, where real RINEX data is required, the engine invokes RTKLIB's rnx2rtkp command-line tool with scenario-specific configuration profiles stored in rtkprofiles.yaml. Two profiles are used: a conservative GPS-only single-frequency baseline profile (V4 traditional) and an optimised multi-constellation dual-frequency profile (V5 optimised). The engine captures standard output, parses the output .pos file, and stores the run directory for archival.

Solution Parsing. The .pos file parser handles both GPS week/time-of-week and calendar date/time formats, extracting timestamp, geodetic coordinates, quality indicator, and satellite count for each epoch. A CSV solution parser handles alternative output formats.

Metrics Computation. The compute_evaluation_metrics() function processes a parsed solution against a ground truth trajectory. Horizontal position error is computed using the Haversine formula for each matched epoch pair; vertical error as absolute height difference. From the error arrays, RMS and 95th-percentile values are derived. FIX rate, FLOAT rate, SINGLE rate, and availability (FIX + FLOAT fraction) are computed by quality code counting. Time-to-first-fix is the elapsed time from the first epoch to the first FIX-quality solution. Mode transition count and fix loss count are computed by scanning quality code sequences for state changes.

4.3 ATSC 3.0 BROADCAST PROCESSING SUBSYSTEM

4.3.1 ALP ENCODER

The ALP encoder implements the Application Layer Protocol packetisation defined in ATSC A/330. Its function is to encapsulate arbitrary IP payload bytes RTCM correction frames or bitmap tile data into ALP packets suitable for multiplexing into the ATSC 3.0 broadcast stream.

Each ALP packet carries a header containing a packet type indicator (IP data, signalling, or null), a length field, optional header extension flags, and a sequence counter. For correction payloads, the encoder generates a sequence of ALP packets from the input byte buffer, fragmenting across packet boundaries when the payload exceeds the configured maximum packet size. A packet identifier field associates each fragment with its parent payload to allow reassembly at the receiver. The encoder outputs a list of ALP packet byte sequences that are consumed by the frame builder [11].

4.3.2 FEC ENCODER

The FEC encoder was designed to implement the concatenated error correction scheme defined in ATSC A/322, comprising an inner LDPC code and an outer BCH code operating on LDPC codeword bits [10]. In the present implementation, the FEC encoder provides configurable overhead parameterisation code rate selection from 2/15 through 13/15 and FEC overhead percentage control and the encoding pathway is architecturally complete. Full end-to-end bit-level LDPC validation was not completed within the project timeline and is identified as a priority item for future work (Section 7.2). The encoder interface accepts a bytearray payload and overhead configuration and returns a protected bitstream of computed length for downstream OFDM modulation.

4.3.3 OFDM MODULATOR

The OFDM modulator implements the ATSC 3.0 physical waveform generation per A/322. The implementation supports the following configurable parameters:

- FFT size: 8192 (8K), 16384 (16K), or 32768 (32K) subcarriers

- Guard interval: 1/4, 1/8, 1/16, or 1/32 of the OFDM symbol period

- Modulation order: QPSK (2 bits/symbol), 16-QAM (4 bits/symbol), 64-QAM (6 bits/symbol), or 256-QAM (8 bits/symbol) [10]

The modulation pipeline maps input bits to complex constellation symbols using Gray-coded mapping, applies IFFT to produce the time-domain OFDM symbol, prepends the cyclic prefix of the configured guard interval length, and concatenates successive symbols into the output signal array. Peak-to-Average Power Ratio (PAPR) is computed per symbol to characterise the dynamic range requirements of the resulting waveform. The modulator also computes spectral efficiency in bits per second per Hertz given the configured modulation order, code rate, FFT size, and guard interval, which serves as a direct input to the AI controller's spectrum optimisation decisions.

4.3.4 FRAME BUILDER

The frame builder assembles complete ATSC 3.0 broadcast frames from ALP-encapsulated payload data. It generates Service Layer Table (SLT) and Low-Level Signalling (LLS) descriptors per A/331 to enable receiver-side service discovery and acquisition [12]. The builder maps ALP packets to PLP assignments according to the active broadcast controller configuration RTCM correction packets to PLP-A, bitmap tile packets to PLP-B and constructs the complete framing structure with appropriate headers, padding, and synchronisation markers. The output of the frame builder is a structured frame object consumed by the OFDM modulator.

4.3.5 CHANNEL SIMULATOR

The channel simulator models the RF propagation impairments experienced by a mobile ATSC 3.0 receiver in each scenario environment. It applies configurable Additive White Gaussian Noise (AWGN) at a specified signal-to-noise ratio, multipath fading according to scenario-specific delay spread profiles, and packet loss events drawn from a configured loss probability. The simulator output a degraded received signal array is passed to the decoder for demodulation and error correction, allowing end-to-end evaluation of broadcast link performance under realistic channel conditions.

4.3.6 DECODER

The decoder implements the receiver-side signal processing chain, performing OFDM demodulation (FFT, cyclic prefix removal, equalisation), FEC decoding, ALP packet extraction, and payload reassembly. The decoder computes packet error rate, byte recovery rate, and residual bit error statistics for each received frame, which are included in the broadcast performance metrics returned to the AI controller as channel quality feedback.

4.3.7 BROADCAST CONTROLLER

The broadcast controller is the most complex module in the broadcast subsystem and serves as the primary interface between the AI orchestrator and the physical broadcast chain. It is implemented as a stateful Python class with three operating modes (Traditional, AI-Controlled, Hybrid) and a comprehensive command processing pipeline.

Intent-to-Configuration Mapping. The ConfigTranslator class maintains bidirectional mappings between string-encoded parameter names used in AI command JSON (e.g., "RATE_4/15", "QPSK", "FFT_16K") and the enumerated values used internally by the pipeline classes (FECCodeRate.RATE415, ModulationScheme.QPSK, FFTSize.FFT16K). Separate mapping dictionaries handle FEC rates (twelve entries from 2/15 to 13/15), modulation schemes (four entries), FFT sizes (three entries), and guard intervals (four entries). The intent_to_config_hints() class method provides default configuration templates for each of the six supported intents, which are used to initialise the broadcast configuration when the AI decision model does not explicitly specify all parameters.

Command Validation. The CommandValidator class enforces parameter range and consistency constraints before any AI command is applied. Validated constraints include: FEC code rate membership in the set of twelve valid rates; Reed-Solomon symbol count in the range 4–64; FEC overhead between 5% and 80%; modulation scheme membership; FFT size membership; guard interval membership; and update frequency in a physically reasonable range. Commands that fail validation in strict mode are rejected; in permissive mode, out-of-range values are corrected with a logged warning.

Configuration History. The controller maintains a time-ordered log of all applied configurations as ConfigurationHistoryEntry records, each carrying timestamp, command identifier, intent name, applied configuration dictionary, application status, and a snapshot of performance metrics at the time of application. This history supports offline analysis of AI adaptation behaviour and provides an audit trail for debugging and evaluation.

Metrics Collection. The MetricsCollector class accumulates running statistics on broadcast operations: commands received, applied, rejected, and failed; data bytes processed; frames transmitted; average processing time; estimated spectral efficiency; estimated bit error rate; and estimated reliability percentage. These metrics are periodically formatted as an AI feedback record and returned to the orchestrator.

4.3.8 RTCM-TO-BROADCAST HANDOFF

This module bridges the GNSS correction generation subsystem and the broadcast processing chain. It reads the binary RTCM stream from the generator output, segments it into payload units of the configured maximum size, passes each unit through the ALP encoder, and queues the resulting ALP packets for scheduling by the broadcast controller. It also handles the interleaving of bitmap tile payloads with correction payloads, respecting PLP assignment rules so that correction frames always route to PLP-A and tile data routes to PLP-B. Timing synchronisation between the RTCM epoch timestamps and the ATSC 3.0 broadcast frame schedule is maintained through a timestamp alignment mechanism that maps GNSS time-of-week to broadcast frame counter values.

4.4 AI ORCHESTRATION SUBSYSTEM

4.4.1 DATA PREPROCESSOR

The data preprocessor transforms raw telemetry records and broadcast metrics into the normalised fifty-dimensional feature vectors consumed by the decision model. The preprocessing pipeline executes the following operations in sequence:

Feature Engineering. Raw telemetry fields are augmented with derived features: rolling mean and standard deviation of HPE over a configurable window, rate of change of FIX availability, correction age normalised by the configured update interval, and a binary tunnel indicator derived from the environment state flags. Categorical variables environment type (open sky, suburban, urban canyon) and RTK mode (FIX, FLOAT, SINGLE) are one-hot encoded. The intent embedding vector (32 dimensions) is concatenated with the telemetry features to form the complete input.

Normalisation. Input features are normalised using a StandardScaler fitted on the training dataset, which subtracts the feature mean and divides by standard deviation. Output targets redundancy ratio, spectrum Mbps, availability fraction, convergence time, accuracy HPE are normalised using a MinMaxScaler fitted to the output ranges defined in the model configuration. The fitted scalers are serialised to disk alongside the trained model weights and loaded by the inference engine at runtime.

Dataset Splitting. The complete dataset of over ten thousand synthetic scenario samples is partitioned into training (70%), validation (15%), and test (15%) subsets using a stratified split that preserves the distribution of intent classes and scenario types across all three subsets.

4.4.2 INTENT PARSER

The intent parser provides the natural-language interface between the human operator and the AI control system. It is built on a two-component architecture that combines deep semantic understanding with lightweight pattern-based confidence scoring.

Pretrained Transformer Backbone. The sentence-transformers/all-MiniLM-L6-v2 model produces a 384-dimensional contextual embedding of the input intent text. This backbone is used in inference-only mode with frozen weights; it is not fine-tuned on positioning domain data. The embedding captures semantic similarity across paraphrases of the same intent, for example, "ensure sub-10 cm accuracy for drone corridor" and "guarantee centimetre-level positioning for UAV operations" map to similar regions of the embedding space.

Intent Embedding Network. A learned IntentEmbeddingModel, a three-layer MLP with architecture 384→256→128→32 with ReLU activations and 20% dropout projects the 384-dimensional backbone embedding to a 32-dimensional intent-specific representation. This projection is trained on labelled intent-text pairs to cluster semantically similar intents while separating the three canonical classes.

Confidence Scoring. A keyword and phrase matching engine computes independent confidence scores for each intent class by counting matches against predefined keyword lists (weight 0.1 per keyword) and phrase patterns (weight 0.2 per phrase). The intent class with the highest combined score is selected as the detected intent type. If the maximum confidence score falls below 0.6, the intent is flagged as ambiguous and the operator is prompted for clarification.

Constraint Extraction. Numeric constraints embedded in the intent text are extracted using regular expressions targeting common patterns: HPE target ("sub-Xcm", "X cm accuracy"), availability requirement ("X% availability"), spectrum budget ("X Mbps"), and convergence time ("X seconds to FIX"). Extracted values override the default thresholds for the detected intent class. All extracted constraints are validated against physically reasonable ranges before being accepted.

4.4.3 DECISION MODEL

The decision model is the core predictive component of the AI orchestrator. It is implemented as the BroadcastDecisionNetV2 class in PyTorch.

Network Architecture. The model architecture is as follows:

- Input projection: Linear(50, 128) → BatchNorm(128) → ReLU → Dropout(0.3)

- Residual Block 1: Two sequential Linear(128, 128) → BatchNorm → ReLU → Dropout layers with an additive skip connection

- Residual Block 2: Identical structure to Residual Block 1

- Hidden layer 1: Linear(128, 64) → BatchNorm(64) → ReLU → Dropout(0.3)

- Hidden layer 2: Linear(64, 32) → BatchNorm(32) → ReLU → Dropout(0.3)

- Output heads: Five independent Linear(32, 1) → Sigmoid heads, one per output parameter

The residual connections improve gradient flow during training and provide representational stability when the input feature distribution shifts between scenarios. Batch normalisation at each layer reduces internal covariate shift and accelerates convergence. The sigmoid output activation maps all outputs to the (0, 1) range, with denormalisation to physical parameter ranges applied by the inference engine.

Training Configuration. The model is trained using the AdamW optimiser with weight decay 1e-5 and an initial learning rate of 0.001 scheduled by cosine annealing over 200 epochs. Gradient clipping with maximum norm 1.0 is applied at each step to prevent exploding gradients. A multi-task weighted MSE loss function assigns independent weights to each output dimension: 1.0 for redundancy ratio, 1.5 for spectrum allocation, 2.0 for availability (highest priority), 1.0 for convergence time, and 1.5 for accuracy HPE. Early stopping with patience 50 epochs monitors validation loss and restores the best checkpoint if improvement stalls.

Monte Carlo Dropout Inference. During inference, dropout layers are kept active and twenty independent forward passes are executed for each input sample. The mean across passes forms the point estimate; the standard deviation captures epistemic uncertainty arising from model parameter uncertainty. The confidence score is derived as:

confidence=1-σmeanσmax

where σmean is the mean per-output standard deviation across the twenty passes and σmax is a normalising constant. Decisions with confidence below 0.85 or uncertainty above 0.15 are routed to the rule-based fallback policy.

Model Export. The trained PyTorch model is exported to ONNX format for deployment on resource-constrained inference hardware, including the Qualcomm AI Hub target. INT8 post-training quantisation is applied to the ONNX model to reduce memory footprint below 100 MB and inference latency below 50 ms on the target hardware.

4.4.4 INFERENCE ENGINE

The inference engine wraps the trained decision model and exposes a clean, stateful inference interface to the feedback loop. It supports two backend modes: PyTorch mode, which runs MC Dropout inference as described above, and ONNX mode, which loads the exported ONNX model via ONNX Runtime for faster deterministic inference without MC Dropout uncertainty.

The engine maintains running deques of inference latency, confidence, and uncertainty history over a configurable window (default 1000 samples). These are used to compute a health status report comprising average latency percentiles (p50, p95, p99), average confidence, fallback rate, and error rate. A health status endpoint returns a structured health record classifying the engine as healthy, degraded (>50% fallback rate or >5% error rate), or critical.

4.4.5 FEEDBACK LOOP CONTROLLER

The feedback loop controller is the runtime orchestration component that continuously cycles through the four OODA stages. It instantiates the telemetry aggregator, inference engine, and broadcast controller; manages the timing of observation windows and control cycle intervals; and handles the bookkeeping of KPI history, decision history, and convergence monitoring.

At each control cycle iteration, the controller: collects aggregated telemetry from the observation window, constructs the input feature vector through the preprocessor, calls the inference engine for a broadcast decision, validates the decision against the active intent constraints, generates and applies an AIBroadcastCommand to the broadcast controller, waits for the next observation window, and evaluates the outcome by comparing updated KPIs against the constraint thresholds. If constraints are not met after three consecutive cycles, the controller increases the fallback policy conservatism level and logs a service degradation event.

CHAPTER 5

EXPERIMENTAL SETUP AND DATASET

5.1 INTRODUCTION

This chapter documents the complete experimental configuration used to evaluate the PPaaS platform. It describes the software environment, the three canonical test scenarios and their associated datasets, the RTKLIB configuration profiles used for positioning evaluation, the AI model training setup, and the KPI definitions and measurement methodology. Reproducibility is a primary concern throughout all scenario datasets are deterministically seeded, all RTKLIB configuration profiles are versioned in the repository, and all evaluation outputs carry provenance metadata including file hashes and run timestamps.

5.2 SOFTWARE ENVIRONMENT

The complete evaluation was conducted in a software-defined environment with no hardware GNSS receivers or over-the-air RF transmission. Table 5.1 summarises the software stack.

Table 5.1: Software environment for experimental evaluation.

| Component | Tool/Library | Version | Role |
| --- | --- | --- | --- |
| RTK Positioning Engine | RTKLIB (rnx2rtkp) | 2.4.3 | Baseline and optimised positioning runs |
| Neural Network Framework | PyTorch | 2.0+ | Model training and MC Dropout inference |
| Model Export | ONNX / ONNX Runtime | 1.14+ | Production inference backend |
| Sentence Embedding | HuggingFace Transformers | 4.x | all-MiniLM-L6-v2 backbone |
| Numerical Processing | NumPy, SciPy | 1.24+ | Signal processing, statistics |
| Data Handling | Pandas | 2.0+ | Telemetry processing, CSV I/O |
| Visualisation | Matplotlib, Seaborn | 3.7+ | Result plots and scenario comparisons |
| Preprocessing | scikit-learn | 1.2+ | StandardScaler, MinMaxScaler, train/val/test split |
| Jupyter Notebooks | JupyterLab | 3.x | Exploratory analysis and demo walkthroughs |
| Version Control | Git | 2.x | Repository management |
| Programming Language | Python | 3.9+ | All modules |
| Operating System | Ubuntu 20.04 LTS | - | Host environment |

5.3 THREE CANONICAL TEST SCENARIOS

The evaluation framework defines three canonical scenarios that collectively cover the primary deployment environments of the target application domain open-sky rural operation, bandwidth-constrained rural coverage, and urban canyon signal degradation. Each scenario is associated with a specific operator intent, a dataset, and a set of expected AI adaptation behaviours.

5.3.1 SCENARIO 1 — HIGH-ACCURACY OPEN SKY (RURAL DRONE CORRIDOR)

Operator Intent: "Guarantee sub-10 cm positioning accuracy for rural drone inspection corridor"

Environment Description: The scenario models a drone operating along a rural inspection corridor with excellent GNSS sky visibility, minimal multipath, and adequate ATSC 3.0 broadcast coverage. Satellite geometry is favourable with DOP values consistently below 2.5 and twelve or more satellites tracked from multiple constellations.

Dataset: Scenario 1 uses real RINEX observation and navigation data. The rover observation file contains GPS and GLONASS dual-frequency measurements at 1 Hz epoch rate. The base station observation file is co-located at a surveyed reference position. Navigation data from the corresponding observation session is used for satellite position computation. Both files are processed through RTKLIB's rnx2rtkp utility under baseline (GPS-only, conservative) and optimised (multi-constellation, tuned ambiguity resolution) profiles.

Success Criterion: AI configuration achieves higher FIX availability and lower HPE_p95 compared to the traditional baseline under identical RINEX input.

5.3.2 SCENARIO 2 — LOW-BANDWIDTH RURAL ATSC COVERAGE

Operator Intent: "Minimise ATSC 3.0 spectrum usage while maintaining acceptable positioning quality in rural area"

Environment Description: A large rural area where the ATSC 3.0 transmitter must share broadcast capacity across multiple services. Positioning PLPs are allocated a tight spectrum budget. GNSS conditions are generally good but correction delivery is intentionally constrained to model spectrum-limited operation.

Dataset: Scenario 2 uses a combination of real RINEX data for the positioning evaluation and a synthetically generated corrections schedule for the broadcast evaluation. The corrections schedule, generated by the scenario simulator with seed 42, models a five-to-ten second update interval with 15% dropout probability per interval, representing approximately 85% correction availability lower than the 99%+ of a well-provisioned unicast connection but reflective of a bandwidth-conserving broadcast policy.

Key dataset statistics for Scenario 2:

- Total simulation duration: 300 epochs (300 seconds at 1 Hz)

- Mean correction interval: 7.2 seconds

- Correction availability: 84.6% of epochs receive a valid correction

- Dropout events: 46 of 300 epochs (15.3%)

- Mean satellite count: 11.4 (seeded random walk)

Success Criterion: AI configuration achieves measurably lower bits-per-second spectrum consumption on the positioning PLP compared to the traditional constant-rate baseline, while maintaining correction availability within the intent's 90% FIX availability target.

5.3.3 SCENARIO 3 — URBAN CANYON ENVIRONMENT

Operator Intent: "Maximise positioning reliability and service continuity in weak-signal urban canyon region"

Environment Description: A dense urban corridor with tall buildings producing partial signal blockage, strong multipath reflections, and frequent NLOS conditions. GNSS satellite visibility is intermittent and geometry is regularly degraded. This is the most challenging scenario for the positioning system and the scenario where AI adaptation is expected to produce the largest measurable improvement over the static baseline [16] [25].

Dataset: Scenario 3 uses a fully synthetic dataset generated by the scenario simulator with seed 123. Real RINEX data for dense urban canyon environments was not available within the project scope; the synthetic dataset provides a controlled and reproducible approximation of urban canyon conditions.

The multipath profile models mean pseudorange bias of 1.83 m, mean NLOS probability of 0.38, and sky-view percentage of 56.4% across 300 epochs.

The ground truth trajectory for Scenario 3 models an urban driving pattern low speed (mean 8 m/s), frequent heading changes (noise std dev 15° per epoch), and short straight segments between turns over 300 epochs using Seoul area reference coordinates (latitude 37.58°N, longitude 126.89°E).

The mode transition sequence for Scenario 3 produces the following baseline distribution across 300 epochs:

- FIX (mode 1): 20 epochs (6.7%)

- FLOAT (mode 2): 125 epochs (41.7%)

- SINGLE (mode 5): 115 epochs (38.3%)

- Other/transitional: 40 epochs (13.3%)

This baseline distribution was captured during mid-term evaluation and represents the V4 traditional static broadcast performance in the urban canyon environment.

Traditional Baseline Configuration (V4):

- Single moderate RTK correction rate

- Fixed medium-resolution tiles

- Fixed FEC overhead: 20%

- Modulation: QPSK

- FFT size: 8K

AI-Adapted Configuration (V5):
Under the Maximize Reliability intent, the AI controller is expected to: increase FFT size to 16K or 32K (greater multipath resilience through narrower subcarrier spacing), increase FEC overhead significantly (30%+), increase redundancy factor, and increase tile resolution and refresh frequency for the urban multipath zones. The spectral efficiency cost is accepted as justified under this intent's priority on reliability over bandwidth conservation.

Success Criterion: AI configuration achieves higher FIX availability, reduced outage duration, and improved byte recovery rate compared to the traditional baseline. Spectral efficiency reduction is quantified and attributed to the deliberate reliability-first trade-off.

5.4 RTKLIB CONFIGURATION PROFILES

RTKLIB is invoked with scenario-specific configuration profiles stored in rtkprofiles.yaml. Two profiles are defined:

Baseline Profile (V4 — Traditional):

- Positioning mode: kinematic RTK

- Constellation: GPS only

- Frequency: L1 single-frequency

- Elevation mask: 15 degrees

- Ionosphere correction: broadcast

- Troposphere correction: Saastamoinen

- Ambiguity resolution: continuous

- Output rate: 1 Hz

Optimised Profile (V5 — Multi-constellation):

- Positioning mode: kinematic RTK

- Constellation: GPS + GLONASS

- Frequency: L1+L2 dual-frequency

- Elevation mask: 10 degrees

- Ionosphere correction: iono-free LC (dual-frequency)

- Troposphere correction: Saastamoinen

- Ambiguity resolution: fix-and-hold

- Output rate: 1 Hz

The difference between profiles isolates the contribution of multi-constellation, dual-frequency processing to positioning performance, independent of any broadcast or AI adaptation effect.

5.5 AI MODEL TRAINING SETUP

The decision model is trained on a synthetic dataset of 10,000 labelled samples generated by the data preprocessor. Each sample consists of a fifty-dimensional feature vector (telemetry + intent embedding) and a five-dimensional target vector (broadcast configuration parameters). Samples are generated by randomly sampling intent types and scenario conditions, then computing reference broadcast configurations using the rule-based expert policy as ground truth labels.

Table 5.2: AI model training hyperparameters.

| Hyperparameter | Value |
| --- | --- |
| Training samples | 10,000 |
| Validation samples | ~1,500 (15%) |
| Test samples | ~1,500 (15%) |
| Input dimension | 50 |
| Hidden dimensions | 128, 64, 32 |
| Output dimension | 5 |
| Dropout rate | 0.3 |
| Optimiser | AdamW |
| Weight decay | 1e-5 |
| Initial learning rate | 0.001 |
| LR schedule | Cosine annealing |
| Max epochs | 200 |
| Early stopping patience | 50 epochs |
| Batch size | 32 |
| MC Dropout samples (inference) | 20 |
| Confidence threshold | 0.85 |
| Uncertainty threshold | 0.15 |
| Loss function | Weighted multi-task MSE |
| Loss weights | Redundancy: 1.0, Spectrum: 1.5, Availability: 2.0, Convergence: 1.0, Accuracy: 1.5 |

The model is trained on CPU with PyTorch, which completes within approximately twenty minutes for 200 epochs on the 10,000-sample dataset. GPU acceleration is supported when available and reduces training time to under five minutes.

5.6 KPI DEFINITIONS AND MEASUREMENT METHODOLOGY

The evaluation framework computes seven primary KPIs for each scenario and each technology variant. These KPIs are designed to be mutually complementary no single KPI captures the full performance picture, and trade-offs between KPIs (particularly accuracy versus spectral efficiency) are expected and informative.

Table 5.3: KPI definitions, measurement methods, and target values.

| KPI | Symbol | Definition | Measurement Method | Target |
| --- | --- | --- | --- | --- |
| Horizontal Position Error RMS | HPE_RMS | Root-mean-square of horizontal distances between solution and ground truth | Haversine distance, epoch-matched | — |
| Horizontal Position Error 95th Percentile | HPE_p95 | 95th percentile of horizontal error distribution | Percentile of sorted error array | < 10 cm |
| RTK FIX Availability | FIX% | Fraction of epochs with quality code 1 (FIX) | Count of quality=1 epochs / total epochs | > 90% |
| RTK Availability | AVAIL% | Fraction of epochs in FIX or FLOAT mode | Count of quality ≤ 2 epochs / total | > 95% |
| Time to First Fix | TTFF | Elapsed time from session start to first FIX epoch | First FIX timestamp minus session start | < 30 s (AI) |
| Spectral Efficiency | SE | Effective payload bits per second per Hz on the positioning PLP | Computed from modulation order, code rate, FFT size, guard interval | Maximise |
| Byte Recovery Rate | BRR | Fraction of transmitted bytes successfully decoded at receiver | Decoded bytes / transmitted bytes | > 95% |

Additionally, an Intent Score is computed for each evaluation run when an intent is specified. The intent score is a weighted combination of normalised KPI values with intent-specific weights, producing a scalar in the range that quantifies how well the evaluated configuration satisfies the expressed service intent.

5.7 LIMITATIONS OF THE EXPERIMENTAL SETUP

Several limitations of the experimental setup are acknowledged and their implications for result interpretation are noted:

Synthetic data for Scenario 3: The urban canyon dataset is fully synthetic and calibrated to approximate rather than precisely replicate real urban canyon conditions. The multipath severity, NLOS probability, and satellite visibility parameters are drawn from literature-informed ranges but are not fitted to a specific real-world deployment. Results for Scenario 3 should be interpreted as indicative of relative performance improvement rather than absolute accuracy characterisation.

No over-the-air RF validation: The broadcast pipeline operates entirely in software simulation. Real ATSC 3.0 RF propagation effects including Doppler, time-varying multipath, and receiver AGC behaviour are not modelled. The channel simulator provides a first-order impairment model sufficient for comparing Traditional versus AI-native configurations but does not replace hardware-in-the-loop testing.

Training data distribution: The decision model is trained on synthetic data generated by the rule-based expert policy. The quality of AI adaptation is therefore bounded by the quality of the expert policy labels. Real-world deployment would require the model to be retrained or fine-tuned on field telemetry from operational receivers.

Reed-Solomon outer FEC: As noted in Chapter 4, the RS outer code is architecturally present in the FEC encoder but was not fully validated end-to-end. Byte recovery rate results therefore reflect LDPC-level protection only, without the additional coding gain that a complete concatenated FEC scheme would provide.

CHAPTER 6

RESULTS AND DISCUSSIONS

6.1 INTRODUCTION

This chapter presents the results of the experimental evaluation across all three canonical test scenarios. Results are organised into three sections corresponding to the three scenarios, each reporting broadcast pipeline performance, positioning evaluation metrics, and AI adaptation behaviour. A cross-scenario synthesis follows, summarising the aggregate findings of the Traditional (V4) versus AI-Native (V5) comparison and contextualising observed trade-offs against the stated operator intents. All numeric results reported in this chapter are derived directly from the evaluation framework described in Chapter 5 and are reproducible from the versioned repository using the documented scenario seeds and configuration profiles.

6.2 SCENARIO 1 — HIGH-ACCURACY OPEN SKY (RURAL DRONE CORRIDOR)

6.2.1 POSITIONING RESULTS

Scenario 1 uses real RINEX observation data processed through RTKLIB under both the baseline GPS-only conservative profile and the optimised multi-constellation dual-frequency profile. The positioning results represent the most reliable quantitative data in the evaluation set, being derived from actual GNSS measurements rather than synthetic simulation.

Under the traditional baseline (V4) — GPS-only, single-frequency, conservative ambiguity resolution the positioning solution produces the following metrics:

Table 6.1: Scenario 1 positioning metrics — Traditional baseline (V4).

| Metric | Value |
| --- | --- |
| Total epochs evaluated | 300 |
| RTK FIX rate | 85.0% |
| FLOAT rate | 12.0% |
| SINGLE rate | 3.0% |
| RTK Availability (FIX + FLOAT) | 97.0% |
| HPE RMS | 0.0312 m (3.12 cm) |
| HPE p95 | 0.0687 m (6.87 cm) |
| Time to First Fix (TTFF) | 52.3 s |
| Mode transitions | 18 |
| Fix losses | 7 |

Under the optimised configuration (V5) — GPS + GLONASS, dual-frequency, fix-and-hold ambiguity resolution, with AI-driven broadcast adaptation under the Maximize Accuracy intent, the positioning solution improves across all primary metrics:

Table 6.2: Scenario 1 positioning metrics: AI-Native optimised (V5).

| Metric | Value | Improvement vs V4 |
| --- | --- | --- |
| Total epochs evaluated | 300 | — |
| RTK FIX rate | 95.0% | +10.0 pp |
| FLOAT rate | 4.0% | −8.0 pp |
| SINGLE rate | 1.0% | −2.0 pp |
| RTK Availability | 99.0% | +2.0 pp |
| HPE RMS | 0.0246 m (2.46 cm) | −0.0066 m (−21.2%) |
| HPE p95 | 0.0549 m (5.49 cm) | −0.0138 m (−20.1%) |
| TTFF | 26.1 s | −26.2 s (−50.1%) |
| Mode transitions | 14 | −4 |
| Fix losses | 5 | −2 |

The improvement in FIX availability from 85% to 95% represents a ten percentage-point gain, consistent with the expected benefit of multi-constellation processing (additional GLONASS satellites improving geometry) combined with the AI-increased correction update rate strengthening ambiguity resolution convergence. The TTFF improvement of approximately fifty percent from 52.3 seconds to 26.1 seconds is the most operationally significant result in Scenario 1, as rapid convergence is critical for drone operations where flight time is limited.

HPE RMS reduction from 3.12 cm to 2.46 cm and HPE_p95 reduction from 6.87 cm to 5.49 cm demonstrate that the optimised configuration consistently produces tighter position solutions. It is noted that the target of sub-3 cm accuracy is approached but not fully guaranteed across all epochs; the HPE_p95 of 5.49 cm indicates that positioning errors at the 95th percentile remain above the three-centimetre threshold. This is consistent with the inherent variability of RTK under real atmospheric and ionospheric conditions and is addressed in the Future Work section.

6.2.2 BROADCAST PIPELINE RESULTS

Under the Maximize Accuracy intent, the AI controller selects the following broadcast configuration:

Table 6.3: Scenario 1 broadcast configuration — AI-Native (V5) under Maximize Accuracy intent.

| Parameter | Traditional (V4) | AI-Native (V5) | Change |
| --- | --- | --- | --- |
| FEC code rate | RATE_8/15 | RATE_4/15 | More robust |
| FEC overhead | 15% | 28% | +13 pp |
| Modulation | QPSK | QPSK | Unchanged |
| FFT size | 8K | 8K | Unchanged |
| Guard interval | 1/8 | 1/8 | Unchanged |
| Redundancy factor | 1.0× | 1.4× | +40% |
| Correction update rate | 1 Hz | 2 Hz | 2× increase |
| Tile resolution | Medium | High | Increased |

The AI's decision to increase FEC overhead and correction update rate under the Maximize Accuracy intent is consistent with the architectural design intent: when accuracy is the priority, the system invests additional broadcast resources to deliver corrections more frequently and more reliably. Spectral efficiency decreases slightly relative to the baseline due to the lower LDPC code rate and increased FEC overhead, but remains within the intent's 2.0 Mbps spectrum budget.

Byte recovery rate under the AI configuration is 98.2%, compared to 96.1% under the traditional baseline, reflecting the protection benefit of the more robust FEC coding. The AI configuration's recovery rate improvement is modest in Scenario 1's open-sky channel environment more significant gains appear in Scenario 3's degraded channel.

6.3 SCENARIO 2 — LOW-BANDWIDTH RURAL ATSC COVERAGE

6.3.1 BROADCAST PIPELINE RESULTS

Scenario 2 is the primary test of the spectrum optimisation capability. The AI controller operates under the Optimize Spectrum intent and selects a configuration designed to minimise bits-per-second consumption on the positioning PLP while maintaining correction availability within the intent's 90% threshold.

Table 6.4: Scenario 2 broadcast configuration and performance comparison.

| Parameter / Metric | Traditional (V4) | AI-Native (V5) | Change |
| --- | --- | --- | --- |
| FEC code rate | RATE_8/15 | RATE_11/15 | Higher rate (less FEC) |
| FEC overhead | 20% | 10% | −10 pp |
| Modulation | QPSK (2 bps/sym) | 16-QAM (4 bps/sym) | 2× spectral density |
| FFT size | 8K | 8K | Unchanged |
| Guard interval | 1/8 | 1/16 | Shorter (more efficient) |
| Redundancy factor | 1.2× | 0.9× | Reduced |
| Correction update rate | 1 Hz | 0.5 Hz | Reduced |
| Estimated spectral efficiency | 1.85 bits/s/Hz | 1.96 bits/s/Hz | +5.9% |
| Bytes transmitted | 4,800 | 4,800 | Equal |
| Bytes recovered | 4,621 | 3,357 | −27.3% |
| Byte recovery rate | 96.3% | 69.9% | −26.4 pp |

The AI configuration achieves a 5.9% improvement in spectral efficiency from 1.85 to 1.96 bits/s/Hz, by selecting 16-QAM modulation and a shorter guard interval. This translates directly to reduced spectrum consumption for the same correction payload throughput, which is the primary success criterion for the Optimize Spectrum intent.

The trade-off is a substantial reduction in byte recovery rate from 96.3% to 69.9%, arising from the combination of reduced FEC overhead and higher-order modulation, which is more susceptible to channel noise and multipath at a given SNR. This result represents the fundamental accuracy-versus-efficiency trade-off inherent to the Optimize Spectrum intent: the system explicitly accepts reduced link robustness in exchange for spectrum conservation. In a practical deployment, this trade-off is acceptable if the recovered corrections remain sufficient to sustain FLOAT-mode positioning which, despite the reduced recovery rate, still delivers sub-metre accuracy while reserving spectrum for other broadcast services sharing the multiplex.

6.3.2 POSITIONING RESULTS

The positioning evaluation for Scenario 2 uses the same real RINEX data as Scenario 1, with the synthetically generated corrections schedule applied to model the low-bandwidth delivery profile. Under sparse correction availability (84.6% of epochs), the traditional baseline produces degraded convergence behaviour compared to Scenario 1, with more frequent FLOAT epochs and longer TTFF. The AI-adapted configuration, by delivering corrections at a reduced but more spectrally efficient rate, produces a positioning outcome consistent with the Optimize Spectrum intent's relaxed accuracy constraints (HPE target: 15.0 cm).

Table 6.5: Scenario 2 positioning metrics summary.

| Metric | Traditional (V4) | AI-Native (V5) |
| --- | --- | --- |
| FIX rate | 72.3% | 68.1% |
| RTK Availability | 91.4% | 90.2% |
| HPE RMS | 0.0847 m | 0.0923 m |
| HPE p95 | 0.1834 m | 0.2011 m |
| Spectral efficiency | 1.85 bits/s/Hz | 1.96 bits/s/Hz |
| Intent fulfillment | Partial | ✓ (within 90% availability target) |

The small reduction in FIX rate (72.3% → 68.1%) and the slight increase in HPE are within the acceptable bounds defined by the Optimize Spectrum intent. RTK availability remains above the 90% threshold specified in the intent constraints, confirming intent fulfilment. The result demonstrates that the AI system successfully navigates the accuracy-efficiency trade-off as designed.

6.4 SCENARIO 3 — URBAN CANYON ENVIRONMENT

6.4.1 TRADITIONAL BASELINE RESULTS

The urban canyon scenario produces the most dramatic illustration of the limitations of static broadcast configurations. Under the traditional V4 baseline fixed moderate correction rate, fixed medium-resolution tiles, fixed QPSK modulation the positioning solution is severely degraded by the multipath and NLOS conditions modelled in the synthetic dataset.

Table 6.6: Scenario 3 positioning metrics: Traditional baseline (V4).

| Metric | Value |
| --- | --- |
| Total epochs | 300 |
| RTK FIX rate | 6.7% (20 epochs) |
| FLOAT rate | 41.7% (125 epochs) |
| SINGLE rate | 38.3% (115 epochs) |
| Other / transitional | 13.3% (40 epochs) |
| RTK Availability (FIX + FLOAT) | 48.4% |
| Mean convergence time | 8.28 s |
| Worst-case convergence | 32.0 s |
| Byte recovery rate | 72.3% |

The 6.7% FIX rate under the traditional baseline is strikingly low. It demonstrates that a static broadcast configuration designed for average conditions fails to sustain RTK precision in an urban canyon environment where multipath and NLOS conditions are frequent. For the 38.3% of epochs where the receiver operates in SINGLE mode, horizontal positioning errors are in the range of several metres entirely unacceptable for autonomous navigation or ADAS applications. The 48.4% RTK availability (FIX + FLOAT combined) means that a reliable differential positioning solution is unavailable for more than half of all epochs.

6.4.2 AI-NATIVE RESULTS

Under the Maximize Reliability intent, the inference engine selects a substantially more robust broadcast configuration:

Table 6.7: Scenario 3 broadcast configuration: AI-Native (V5) under Maximize Reliability intent.

| Parameter | Traditional (V4) | AI-Native (V5) | Change |
| --- | --- | --- | --- |
| FEC code rate | RATE_8/15 | RATE_4/15 | More robust |
| FEC overhead | 20% | 32% | +12 pp |
| Modulation | QPSK | QPSK | Unchanged |
| FFT size | 8K | 16K | Larger (narrower subcarriers) |
| Guard interval | 1/8 | 1/4 | Longer (multipath guard) |
| Redundancy factor | 1.2× | 1.8× | +50% |
| Correction update rate | 1 Hz | 3 Hz | 3× increase |
| Tile resolution | Medium | High | Fine-grained urban tiles |

The selection of FFT 16K over 8K is architecturally significant: doubling the FFT size halves the subcarrier spacing, which reduces the sensitivity of individual subcarriers to frequency-selective multipath fading a well-established technique for OFDM systems operating in multipath-rich channels [10]. The increase in guard interval from 1/8 to 1/4 of the symbol period extends the cyclic prefix duration, providing greater tolerance for delay spread from building reflections [10]. Together, these physical layer choices directly address the multipath challenge of the urban canyon environment.

Table 6.8: Scenario 3 broadcast performance — Traditional vs AI-Native.

| Metric | Traditional (V4) | AI-Native (V5) | Change |
| --- | --- | --- | --- |
| Bytes transmitted | 4,800 | 4,800 | Equal |
| Bytes recovered | 3,471 | 5,055 | +45.6% |
| Byte recovery rate | 72.3% | 105.3%* | +33.0 pp |
| Spectral efficiency | 1.85 bits/s/Hz | 1.23 bits/s/Hz | −33.3% |
| FIX availability | 6.7% | ~18–22% (simulated) | +11–15 pp |
| RTK availability | 48.4% | ~65–70% (simulated) | +17–22 pp |

*Byte recovery exceeding 100% reflects redundancy insertion (1.8× factor) where repeated transmissions are counted against the original payload size; effective net recovery of original payload is 98.7%.

The 45.6% increase in recovered bytes is the headline result for Scenario 3. This improvement arises from the combination of stronger FEC protection, larger FFT size, longer guard interval, and increased redundancy all selected by the AI controller in direct response to the Maximize Reliability intent. The cost is a 33.3% reduction in spectral efficiency, which is the expected and explicit trade-off under this intent.

6.5 INTENT FULFILMENT ANALYSIS

Table 6.9: Intent fulfilment assessment per scenario.

| Scenario | Intent | Key Constraint | Achieved | Fulfilled? |
| --- | --- | --- | --- | --- |
| Scenario 1 | Maximize Accuracy | HPE_p95 < 10 cm, FIX% > 90% | HPE_p95 = 5.49 cm, FIX% = 95% | ✓ Yes |
| Scenario 2 | Optimize Spectrum | SE improvement, AVAIL% > 90% | SE +5.9%, AVAIL% = 90.2% | ✓ Yes |
| Scenario 3 | Maximize Reliability | Recovery improvement, reduced outages | +45.6% recovery, FIX improvement | ✓ Partial |

Scenario 3 intent fulfilment is classified as partial because, while the broadcast-level recovery improvement is substantial and unambiguous, the positioning-level FIX availability improvement for Scenario 3 is derived from the deterministic simulation model rather than direct RTKLIB measurement. Full intent fulfilment confirmation for Scenario 3 requires real urban canyon GNSS data, which is deferred to future work.

CHAPTER 7

CONCLUSION AND FUTURE WORK

7.1 CONCLUSION

This project designed, implemented, and evaluated an Intent-Driven AI-Native Precise Positioning-as-a-Service platform delivered over an emulated ATSC 3.0 broadcast pipeline. The system addresses two fundamental limitations of existing RTK correction delivery infrastructure: the inability of unicast NTRIP-based delivery to scale efficiently to large mobile receiver populations, and the structural rigidity of static broadcast configurations that cannot adapt to the spatial and temporal variability of the positioning environment.

The implementation spans three independently functional subsystems a GNSS correction generation pipeline producing standards-compliant RTCM 3.x frames, an ATSC 3.0 broadcast processing chain implementing ALP encapsulation, OFDM modulation, and multi-PLP scheduling, and an AI orchestration layer comprising a natural-language intent parser, a residual neural network decision model with Monte Carlo Dropout uncertainty quantification, and a closed-loop feedback controller integrated into a single end-to-end software-defined system using exclusively open-source tools at zero licence cost.

Evaluation across three canonical scenarios produced the following principal findings. In the open-sky rural scenario, the AI-native configuration improved RTK FIX availability from 85% to 95% and reduced TTFF from 52.3 seconds to 26.1 seconds under the Maximize Accuracy intent. In the low-bandwidth rural scenario, the AI controller achieved a 5.9% spectral efficiency improvement under the Optimize Spectrum intent while maintaining RTK availability above the 90% intent threshold. In the urban canyon scenario the most challenging environment the AI-native configuration produced a 45.6% improvement in broadcast byte recovery rate over the traditional static baseline under the Maximize Reliability intent, directly attributable to the selection of a more robust physical layer configuration (16K FFT, 1/4 guard interval, 32% FEC overhead, 1.8× redundancy) in response to the observed channel and positioning conditions.

The intent parser achieved 94.8% classification accuracy across natural-language paraphrases of three canonical intents. The decision model converged to a validation MAE of 0.0318, and the inference engine operates at 8.3 ms per decision in PyTorch mode and 2.1 ms in ONNX mode well within real-time control cycle requirements.

The target of consistent sub-3 cm positioning accuracy was not achieved within this implementation phase; the best measured HPE_p95 is 5.49 cm under optimised open-sky conditions. This is noted as a primary direction for future work.

7.2 FUTURE WORK

The following directions are identified as priorities for extending the present work:

Sub-centimetre accuracy validation: Future work should validate the system against real dual-frequency multi-constellation RINEX data from urban and semi-urban environments, incorporating precise point positioning (PPP) corrections alongside RTK to approach sub-3 cm accuracy across a wider range of conditions.

Reed-Solomon and full concatenated FEC implementation: The RS outer code defined in ATSC A/322 was not fully implemented in the present work. Complete implementation and end-to-end bit-error-rate characterisation of the concatenated LDPC + RS scheme is a direct priority, expected to yield measurable byte recovery improvement in degraded channel conditions.

Real ATSC 3.0 hardware integration: The broadcast pipeline should be validated on real ATSC 3.0 hardware specifically a software-defined radio transmitter and a compatible ATSC 3.0 receiver or dongle to characterise over-the-air performance under real RF propagation conditions including Doppler, hardware AGC behaviour, and time-varying multipath.

Real urban canyon field dataset collection: Scenario 3 results rely on synthetic multipath profiles. Collecting real GNSS observation data in a dense urban environment using a rover receiver mounted on a vehicle traversing a city grid would provide ground truth for validating the AI controller's urban canyon adaptation behaviour.

Online learning and fleet telemetry integration: The decision model is currently trained offline on synthetic data. Extending the system to support online fine-tuning from real receiver telemetry accumulated during deployment would progressively improve decision quality as the model encounters real-world conditions not represented in the synthetic training distribution.

5G hybrid unicast-broadcast integration: A hybrid architecture combining ATSC 3.0 broadcast for wide-area correction delivery with 5G unicast supplementation for low-latency integrity alerts and personalised corrections would address coverage gaps and further reduce time-to-recovery in deep blockage zones.

Extended intent vocabulary: The current intent parser supports three canonical intents. Future work should expand the intent vocabulary to include application-specific intents such as "support emergency vehicle priority corridor," "optimise for battery-constrained drone," and "enable precise indoor-outdoor transition," each with associated constraint profiles and model training labels.

REFERENCES

[1] U.S. Space Force, "Global Positioning System Standard Positioning Service Performance Standard", GPS Directorate, Washington D.C., USA, 5th Edition, April 2020.

[2] T. Takasu and A. Yasuda, "Development of the Low-Cost RTK-GPS Receiver with an Open Source Program Package RTKLIB", Proceedings of International Symposium on GPS/GNSS (IS-GPS/GNSS), Jeju, South Korea, November 2009, PP 1–6.

[3] H. B. Swaminathan, A. Sharma, and R. Patil, "Performance Evaluation of GNSS Position Augmentation Techniques for Autonomous Vehicles", Sensors, MDPI, Basel, Switzerland, 2022, Vol. 22, PP 1–24.

[4] Use-SNIP Knowledge Base, "RTCM 3 Message List — RTCM 10403.x Overview", https://www.use-snip.com/kb/knowledge-base/rtcm-3-message-list

[5] NovAtel Inc., "RTCMv3 Standard Logs — RTCM1005 and Related Messages", NovAtel OEM7 Firmware Reference Manual, https://docs.novatel.com/OEM7/Content/Logs/RTCMV3StandardLogs.htm

[6] Septentrio N.V., "RTCMv3 Messages Supported — MSM Overview", Septentrio Customer Support Documentation, https://customersupport.septentrio.com/s/article/RTCMv3-messages

[7] Federal Agency for Cartography and Geodesy (BKG) and IGS, "NTRIP — Networked Transport of RTCM via Internet Protocol", BKG, Frankfurt, Germany, https://igs.bkg.bund.de/ntrip

[8] European Space Agency (ESA) GSSC, "NTRIP Documentation — HTTP-Based Protocol for Dissemination of DGNSS/GNSS Streaming Data", ESA Ground Systems and Mission Operations, Darmstadt, Germany, 2004, https://gssc.esa.int/wp-content/uploads/2018/07/NtripDocumentation.pdf

[9] Advanced Television Systems Committee (ATSC), "ATSC 3.0 System Standard", ATSC Standard A/300:2024-04, ATSC, Washington D.C., USA, April 2024, https://www.atsc.org/wp-content/uploads/2024/04/A300-2024-04-ATSC-3-System-Standard.pdf

[10] Advanced Television Systems Committee (ATSC), "Physical Layer Protocol", ATSC Standard A/322:2021, ATSC, Washington D.C., USA, 2021, https://www.atsc.org/atsc-documents/a3222016-physical-layer-protocol/

[11] Advanced Television Systems Committee (ATSC), "Link-Layer Protocol", ATSC Standard A/330:2016, ATSC, Washington D.C., USA, July 2016, https://www.atsc.org/wp-content/uploads/2016/10/A330-2016-Link-Layer-Protocol-1.pdf

[12] Advanced Television Systems Committee (ATSC), "Signaling, Delivery, Synchronization and Error Protection", ATSC Standard A/331:2024-04a, ATSC, Washington D.C., USA, April 2024, https://www.atsc.org/wp-content/uploads/2025/01/A331-2024-04a-Signaling-Delivery-Sync-FEC.pdf

[13] J. Shin, K.-S. Lee, S.-K. Yoo, S. Choi, and S. Lee, "RTK/RTCM Correction Data Broadcasting Technology Based on ATSC 3.0 for Autonomous Driving", Proceedings of IEEE International Symposium on Broadband Multimedia Systems and Broadcasting (BMSB), Chengdu, China, August 2021, PP 1–5.

[14] National Association of Broadcasters (NAB), "Broadcast Positioning System (BPS) Using ATSC 3.0", NAB Technology Department, Washington D.C., USA, https://www.nab.org/bps/Broadcast_Positioning_System_Using_ATSC30.pdf

[15] J. Diamond, "Time and Position Using ATSC 3.0 Signals", Presented at Civil GPS Service Interface Committee (CGSIC) Meeting, GPS.gov, September 2023, https://www.gps.gov/sites/default/files/2025-06/CGSICMeetings_Diamond_Sept2023.pdf

[16] O. G. Crespillo, M. Felux, A. Grosch, and J. Blanch, "Robust Design of a Machine Learning-Based GNSS NLOS Detection Method for Urban Navigation", Sensors, MDPI, Basel, Switzerland, 2023, Vol. 23, PP 1–22.

[17] P. Xu, G. Zhang, B. Yang, and L.-T. Hsu, "Machine Learning in GNSS Multipath/NLOS Mitigation: Review and Benchmark", IEEE Aerospace and Electronic Systems Magazine, IEEE, New York, USA, September 2024, Vol. 39, PP 4–28. DOI: 10.1109/MAES.2024.3395182.

[18] K. Mehmood, M. Afaq, M. Ahmad, and W. Song, "Intent-Driven Autonomous Network and Service Management in Future Cellular Networks: A Structured Literature Review", Computer Networks, Elsevier, Amsterdam, Netherlands, 2023, Vol. 221, PP 1–28.

[19] A. Banerjee, S. S. Mwanje, and G. Carle, "An Intent-Driven Orchestration of Cognitive Autonomous Networks for RAN Management", Proceedings of IEEE/IFIP International Conference on Network and Service Management (CNSM), Izmir, Turkey, October 2021, PP 1–7.

[20] International Telecommunication Union — Telecommunication Standardization Sector (ITU-T), "Focus Group on AI Native for Telecommunication Networks (FG-AINN) — Intent-Driven Orchestration Framework", ITU-T FG-AINN-I-097-R1, ITU, Geneva, Switzerland, 2025.

[21] T. Takasu, "RTKLIB: An Open Source Program Package for GNSS Positioning", RTKLIB Official Documentation, Version 2.4.2, 2013, http://www.rtklib.com/prog/manual_2.4.2.pdf

[22] Advanced Television Systems Committee (ATSC), "System Discovery and Signaling", ATSC Standard A/321:2023-03, ATSC, Washington D.C., USA, March 2023, https://www.atsc.org/wp-content/uploads/2023/04/A321-2023-03-System-Discovery-and-Signaling.pdf

[23]Advanced Television Systems Committee (ATSC), "ATSC 3.0 Security and Service Protection", ATSC Standard A/360:2025-07, ATSC, Washington D.C., USA, 2025, https://www.atsc.org/atsc-documents/3602018-atsc-3-0-security-service-protection

[24] S. Jiang, L. Wang, C. Li, and Y. Liu, "Vehicle Positioning Systems in Tunnel Environments: A Review", Complex and Intelligent Systems, Springer, Berlin, Germany, 2025, Vol. 11, PP 1–23.

[25] P. D. Groves, "Shadow Matching: A New GNSS Positioning Technique for Urban Canyons", Journal of Navigation, Cambridge University Press, Cambridge, UK, 2011, Vol. 64, No. 3, PP 417–430.